[image: image113.jpg]





 OCTE Emphasis Course Resources




 OCTE Emphasis Course Resources


Broad Based Technology:
Manufacturing Technology

Area of Emphasis:

Manufacturing Technology: Welding
TMY4C

Other Applicable Emphasis Courses in Manufacturing Technology:

	TMM3M

TMR3M

TMC3C

TMI3C

TMP3C

TMT3C

TMY3C

TMO3E

TMS3E

TMW3E

TMM4M

TMR4M

TMC4C

TMI4C

TMP4C

TMT4C

TMO4E

TMS4E

TMW4E
	Manufacturing Engineering Technology: Mechanical Engineering

Manufacturing Engineering Technology: Robotics and Control Systems

Manufacturing Technology: Computer Aided Manufacturing

Manufacturing Technology: Industrial Maintenance

Manufacturing Technology: Precision Machining

Manufacturing Technology: Robotics and Control Technician

Manufacturing Technology: Welding Technician

Manufacturing Technology: Machine Operator

Manufacturing Technology: Sheet Metal

Manufacturing Technology: Welding

Manufacturing Engineering Technology: Mechanical Engineering

Manufacturing Engineering Technology: Robotics and Control Systems

Manufacturing Technology: Computer Aided Manufacturing

Manufacturing Technology: Industrial Maintenance

Manufacturing Technology: Precision Machining

Manufacturing Technology: Robotics and Control Technician

Manufacturing Technology: Machine Operator

Manufacturing Technology: Sheet Metal

Manufacturing Technology: Welding


Revision August 2014

This resource was produced
 by the Ontario Council for Technological Education (OCTE),
 in support by the Ministry of Education
 It may be used in its entirety, in part, or adapted.

Table of Contents
Disclaimer
1SECTION 1: GENERAL


1Safe Activity Foundation In Education (Safedoc):: Manufacturing


2Usage Of The Octe Emphasis Course Resources


4Safety Topics For The Classroom


6Emphasis Course Outline And Expectations Examples


7A. Manufacturing Technology


7Fundamentals


7Overall Expectations


7Specific Expectations


8Specific Expectations


9Specific Expectations


11Internet Acceptable Use Agreement Form


12Manu Student Conduct Agreement


13Student Conduct Agreement Form


13Ensure A Safe Workplace


13Prescribed And Non-Prescribed Medications


13Consequences For Improper Action


13SECTION 2: SAFETY DATA SHEETS


13Section Overview


15Air Compressor


16Arc Welding


17Band Saw (1)


18Band Saw (2)


19Band Saw - Horizontal


20Belt and Disk Sander


21Chemical Handling


22CNC Machine


23CNC Lathe


24CNC Milling Machine


25Drill Press


26Radial Arm Drill Press


27Electrical Hazards


28Facility Emergency Procedures


29Fall Protection


30Fire Extinguishers


31First Aid


32First Aid Kits


33Fluid Power Systems


34General Housekeeping


35Grinder – Portable Angle


36Grinder – Bench and Pedestal


37Hand Tools


38Horizontal Milling Machine 1


39Horizontal Milling Machine 2


40Lifting


41Metal Cut Off (Chop) Saw


42Metal Lathe


43Metal Storage & Handling


44Metal Storage & Handling


45Oxy-Acetylene Welding (1)


46Oxy-Acetylene Welding (2)


47Plasma Arc Cutter


48Power Tools (1)


49Power Tools (2)


50Programmable Logic Controllers (Wiring Precautions)


51Robotics


52Sheet Metal Shear/Roller/Bender


53Surface Grinder


54Vertical Milling Machine


55Vise


56Vise


57Welding Operational Safety


59Welding Student Safety


60WHMIS Regulations


60WHMIS Labels


62MSDS Safety Labels


63SECTION 3: SAFETY ASSIGNMENTS AND TESTS


64Safety Assignment # 1 – Room Inventory And Safety Identification


65Safety Assignment # 2 – General Safety


66Safety Assignment # 3 – Perform A Safety Audit


67Welding Hazards Test


67Welding Safety Test


68Welding Fires And Explosions Test


68Welding Fumes And Gases Test


68Containers And Welding Test


69Answers To The Welding Hazards Test


70Answers To Welding Safety Test


70Answers To Welding Fires And Explosions Hazards Test


70Answers To Welding Fumes And Gases Test


71Answers To Containers And Welding Test


73Manufacturing Facilities Health And Safety Inspection Checklist


74Sample Whmis And Msds Quiz


75General Safety Quiz


77General Safety Rules Quiz


78Band Saw Safety Quiz


79Horizontal Bandsaw


79Safety Quiz


80Belt And Disk Sander


80Safety Quiz


81Drill Press Safety Quiz


82Fluid Power Safety Quiz


83Pedestalgrinder Safety Quiz


84Hand Tools Safety Quiz


85Metal Lathe Safety Quiz


86Plc Safety Quiz


87Plasma Arc Cutter Safety Quiz


88Robotics Safety Quiz


89Sheet Metal Tool Safety Quiz


90Surface Grinder Safety Quiz


91Vertical Mill Safety Quiz


92SECTION 4: SAFETY PASSPORTS


93Sample Student Safety Record Card


93Manufacturing Equipment


93Technology Lab Safety Passport


95Safety Passport Form 1


96Safety Passport Form 2


97Safety Passport Form 3


98Arbor Press Passport


99Ergonomics Passport


100Internet Use Passport


101Sample: Record of Safety Training


102WHMIS Regulations


103SECTION 5: EMPHASIS COURSE RESOURCES


105APPENDIX A: HEALTH AND SAFETY RESOURCES


106Health & Safety Ontario (HSO)


109Ontario School Boards Insurance Exchange




APPENDIX B: OCTE SAFETYNET BLANK TEMPLATE ……………………………………….110
Disclaimer

This material was designed to assist teachers implement the Ontario Curriculum – Technological Education (revised Grade 9 -12), but is fully adaptable to the Ontario Curriculum Grade 1 – 8 Science and Technology curriculum. This material was created by members of the Ontario Council for Technology Education (OCTE) subject association and is intended as working guides for classroom, lab or shop activities. Permission is given to reproduce these materials for any purpose except profit. Teachers are encouraged to amend, revise, edit and adapt this material for educational purposes. Please acknowledge the source in all uses. Any references in this document to particular to commercial resources, materials or equipment reflect only the opinions of the writers of this material, and do not reflect any official endorsement by the Ontario Council for Technology Education, the Ontario Ministry of Education, or any other agency or government body.

All materials within these safety related documents are to be considered as suggestions and recommendations only.  These are not legal documents and are not to be considered as legal requirements or as official policy. OCTE or the individual contributors makes no claim to the accuracy or the completeness of the enclosed documents and accepts no responsibility for any damages pertaining to their use. Users of this document should not assume all warnings and precautionary measures are contained herein, that additional information or measures are not required, or that local by-laws, regulations or Board policies are explicitly included.

© Ontario Council for Technology Education 2014
	SECTION 1: GENERAL


Safe Activity Foundation in Education (SAFEdoc):: Manufacturing


In 2013, the SAFEdoc were designed and updated to provide safety data sheets, posters, safety passports, and safety resources for all technology educators. While originally developed as a resource for the Course Profiles, it is available for any grade level or any technology education environment.
In 2012 another resource called the SafetyNET was created by OCTE with many subject-specific exemplars of exciting student projects that incorporate varying levels of safety risk. Please review exemplar resource documents created ‘by teachers for teachers’ with experienced tips and customization options for your course projects at http://www.octelab.com/.
The SAFEdoc were created for eleven separate disciplines per Ontario Ministry Courses:

	Communications, (COM)
	Hospitality and Tourism (HOST)

	Computer Engineering Technology (CET)
	Manufacturing (MANU)

	Construction, Custom Woodworking (CON)
	Technological Design (DESIGN)

	Green Industries (GREEN)
	Transportation (TRANS)

	Hairstyling and Aesthetics (H&A)
	Exploring Technologies (EXPL)

	Health Care (HC)
	


Please note that due to the cross-curricular nature of Technological Education, there may be a need to refer to other SAFEdocs for cross-discipline data sheets. For example, a Computer Engineering teacher may need to utilize construction and manufacturing equipment or communications technology production equipment, and therefore may need to refer to the CON or COM SAFEdoc. Teachers are encouraged to download ALL SAFEdocs for reference at http://octe.on.ca/> Secondary>OCTE lab


This document is a practical safety resource that compliments and elaborates on other recommended resources for technical teachers. See the appendix for linking information such as Live Safe! Work Smart!, the Young Worker’s Awareness Program, and industry associations dedicated to safe working practices.

It is imperative that all students are made aware of the issues of health and safety particular to your class, and that you have assessed and evaluated their understanding before they are allowed to work in a shop environment or on specific procedures or tools. The use of Safety Passports, Safety Agreements, and Safety Tests (provided in this document) is highly recommended. It is also 

Recommended that you look at safety within the project before you move forward (e.g., used flooring being passed through the thickness planer that has floor staples embedded, cutting steel that has a toxic material painted on, etc.)NOTE: While it is important to give students initial safety training and testing at the beginning of the semester, it is also important to practice JIT Safety Training (Just In Time) and to reinforce specific safety procedures and rules each day before initiating new procedures or using equipment. For example, before students use a band saw, review the setup and ask key questions of students before allowing its use.
Usage of the OCTE Emphasis Course Resources
Teachers are encouraged to use and modify this document as they see fit. Individual pages may be directly printed, or custom formatting may be applied for printing any part of the document. General Guidelines may be used in Board or school policy documents. Safety Guidelines may be used as student handouts, as a teacher reference for tests, or printed and mounted as posters around equipment.

The Emphasis Course Resources  also contains sample Safety Passports. These can be used as verification that students have been trained and understand the safety aspects of each equipment or procedure they need to use to accomplish their tasks. There are several formats that may be used. Teachers are encouraged to keep consistent records at all times.

See Appendix A for related safety resources, such as Live Safe, Work Smart; the Young Workers Awareness Program, the Ministry of Labour and other organizations dedicated to safe practices.

It is important that teachers are knowledgeable about their own Board and school policies regarding safety, and that they are familiar with local municipal regulations.

Responsibilities for Safety

[from the Ontario Ministry of Education The Ontario Curriculum(Revised)2009, Technological Education, Grades 9 and 10 (page 28); Grade 11 and 12(page 33)] 
Health and safety is of paramount importance in technological education. In every course, students must be made aware that health and safety is everyone’s responsibility at home, at school, and in the workplace. Before using any piece of equipment or any tool, students must be able to demonstrate knowledge of how the equipment or tool works and of the procedures they must follow to ensure its safe use. Personal protective gear must be worn as required.

Classroom practice and all aspects of the learning environment must comply with relevant municipal, provincial, or federal health and safety legislation, including the following:

· the Ontario Workplace Safety and Insurance Act
· the Workplace Hazardous Materials Information System (WHMIS)
· the Food and Drugs Act
· the Ontario Health Protection and Promotion Act
· the Ontario Building Code
· the Occupational Health and Safety Act
· local by-laws

Teachers should make use of all available and relevant resources to make students sufficiently aware of the importance of health and safety. These resources include:

· Live Safe! Work Smart! – website (http://www.livesafeworksmart.net/) and related resources

· Passport to Safety – website (http://www.passporttosafety.com/ ) and related resources

· Workplace Safety and Insurance Board (WSIB) – website (http://www.wsib.on.ca/splash.html) and related resources

· Industrial Accident Prevention Association (IAPA) – website (http://www.iapa.ca/) and related resources

· Ontario Ministry of Labour (MOL) – website (http://www.labour.gov.on.ca/english/) and related resources

· Canadian Centre for Occupational Health and Safety (CCOHS) – website (http://www.ccohs.ca/) and related resources

· Appropriate Safe Workplace Associations (SWAs) and clinics (http://www.ohscanada.com/links/associations.aspx), such as:

· the Construction Safety Association of Ontario (CSAO) – website (http://www.ihsa.ca/home.cfm) 

· the Ontario Service Safety Alliance (OSSA) – website (http://www.ossa.com/) 

· the Infrastructure Health & Safety Association of Ontario (IHSAO) – website (http://www.ihsa.ca/about/) 

· the Electrical & Utilities Safety Association (E&USA) – website (http://www.eusa.on.ca/)

· the Workers Health & Safety Centre (WHSC) – website (http://www.whsc.on.ca/)

· the Occupational Health Clinics for Ontario Workers (OHCOW) – website (http://www.ohcow.on.ca/)
Teachers should also be aware of the Occupational Health and Safety Act, Regulations 857, Amended to O. Reg. 352/91. The Occupational Health and Safety Act can be found at:

http://www.e-laws.gov.on.ca/html/regs/english/elaws_regs_900857_e.htm 

Delegating the Responsibilities for Safety 


As well, there are key areas of responsibility that must be clearly delegated for all technological subject areas and they must be addressed for their individual board, school and facility. 

These may include administration, department heads, technology teachers, students, board facilities, custodian/maintenance and other local partners or board-defined roles. This information can be found in the OCTE SAFEdocs at www.octe.on.ca.

Safety Perspective Overview









Safety Topics for the Classroom

The following are suggested topics for teaching in the classroom. See Appendix A for available resources pertinent to general safety and particular safety rules and procedures for your subject area. See Appendix B for specific resources or links that are associated with Communications Technology. See also your Board, school and relevant municipal policies for local safety rules and procedures.

Emergency Procedures
procedures for handling fire, security threats, and other emergencies

First Aid
procedures for handling breathing difficulties, bleeding, burns, allergic reactions, epileptic seizures, etc.

Personal Protective Equipment
use of eye, hearing, foot, body, respiratory protection

Ergonomics
safe posture when using equipment, avoiding repetitive stress injuries

Material Handling
procedures for safely handling heavy loads, chemicals, potentially hazardous materials. 
Housekeeping and Storage
procedures and rules regarding maintaining safe facilities and proper storage of materials and equipment

Fire Protection
location and types of fire protection equipment, procedures to follow in the event of a fire or fire alarm

WHMIS
(Workplace Hazardous Materials Identification System)…identification and safe use of hazardous materials

Communication
It is important to the safety of all students and staff at a school that safety be taught and reinforced on a daily basis. Some basic methods of communication are:
· Safety Notice Board, containing posted minutes from the joint health and safety committee and the Occupational Health and Safety Act (must be posted by law)

· visible WHMIS binders, symbols and MSDS sheets

· readily available manuals for the operation of various types machinery, tools or equipment 

· safety posters around major equipment and work areas

· clear and precise instructions, reinforced each time a procedure or equipment is used

· clearly marked areas that contain safety items such as fire extinguishers, eye wash stations, first aid kits, etc.
Emphasis courses provide an opportunity for educators to deliver focused programs to Ontario students who seek to acquire secondary school credits in a specific sector based pathway. Manufacturing and emphasis related courses provide a real world 21st century learning experience for students seeking a career in an emphasis field.  Welding for example, encompass’ a broad range of skills and competencies for students to acquire in addition to addressing the overall expectations and majority of specific core course expectations. This document includes additions to the examples within the specific expectations in order to provide the teacher with a significant understanding of how to address the expectations while providing multiple support resources to deliver activities that are emphasis focused.

The goal is to prepare the student for post-secondary studies in a Manufacturing Welding field, or an entry-level skilled worker with appropriate and adequate safety training, with specific awareness of how to gain further training and safety certifications as needed. 
Each Manufacturing emphasis course provides suggested units of study to help the teacher plan the delivery of the course in an effort to meet the core course expectations through learning activities that impart relevant knowledge and understanding of the emphasis focus.

Emphasis Course Outline and Expectations Examples

The following are related expectations from The Ontario Curriculum 2009 Revised) - Technological Education for TMJ4C: Additional expectation examples have been added that are relevant to the Emphasis area.

Course Description: Welding Emphasis TMY4C
Prerequisite: Any Manufacturing Technology, Grade 11 Course

This course enables students to further develop knowledge and skills related to machining, welding, print reading, computer numerical control (CNC), robotics, and design. Students will develop proficiency in using mechanical, pneumatic, electronic, and computer control systems in a project-based learning environment and may have opportunities to obtain industry-standard training and certification. Students will expand their awareness of environmental and societal issues and career opportunities in the manufacturing industry. This course focus will emphasize student learning in the development of knowledge and skills related to “Welding Principals and Practices” (e.g., safety, processes, print reading, layout, fitting, fabricating and design). 
	                                                                   Safety 

               (Standards, Procedures, Work Habits, Reasonability’s, and Hazard Recognition)

	                                                     Design

     (Design process, Measuring-sketching in isometric, oblique, orthographic and blue print reading and welding symbols.)

	                                            Material Technology 

                   (Metal properties, shapes, alloys, ferrous, none ferrous and Welding Consumables, destructive and non-destructive weld testing, and heat distortion )

	                                                 Equipment

                                       (Identification, Procedures, Safety, Hazards and Certification)

	                                            Welding / Cutting Processes  

                                  (Oxy-fuel, Plasma, SMAW,GMAW, FCW,TIG and Spot Welding)

	                                                  Fabrication

                            (Joints, Positions, Layout, Techniques, and Fitting)


Suggested Units of Study
A. MANUFACTURING TECHNOLOGY 

     FUNDAMENTALS
OVERALL EXPECTATIONS 
By the end of this course, students will: 

A1. demonstrate an understanding of how the design process is used as a problem-solving strategy to develop solutions to challenges or problems in manufacturing technology;
A2. demonstrate an understanding of the processes, tools, and equipment required for making material conversion;
A3. demonstrate a working knowledge of various control systems used in manufacturing processes;
A4. apply relevant mathematical skills, scientific concepts, and technological literacy and communication skills in the study of manufacturing technology.
SPECIFIC EXPECTATIONS

A1l Design Process

By the end of this course, students will:

A1.1 demonstrate proficiency in using a design process to solve manufacturing problems (e.g., how fatigue and weldment failures were due to short cuts taken in the design and manufacturing process)

A1.2 use technical drawings (e.g., orthographic, pictorial, assembly drawings and sketches, exploded views demonstrating part assembly, interpreting and communicating drawings for material preparation for accepting welding beads, metal removal amounts for undercutting material to accept weld prior to machining part surface back to original size; bearing fit, wear plates, mating parts) as part of the design process to develop solutions to challenges or problems in manufacturing technology; 

A2.2 demonstrate a working knowledge of correct tool geometry and terminology when adapting machine tools to allow for specific manufacturing operations (e.g., prepare grinding tools with proper relief angles and cutting edges, use proper tool set-up for the application or using a wobbler in the   drill press to accurately find center to drilling holes, selection of size and grade of welding rod, electrodes, torches etc.) 
B. MANUFACTURING TECHNOLOGY SKILLS
OVERALL EXPECTATIONS 
By the end of this course, students will: 
B1. demonstrate the ability to interpret and prepare technical drawings and develop process plans;
B2. select, prepare, and use appropriate materials when manufacturing a product to meet design specifications;
B3. apply quality control standards when using various advanced metrology tools to measure, lay out, and inspect the design and manufacture of products;

B4. use appropriate tools and equipment and a variety of manufacturing processes when planning and manufacturing a product to meet manufactures specifications.
SPECIFIC EXPECTATIONS
B. MANUFACTURING TECHNOLOGY SKILLS

B. MANUFACTURING TECHNOLOGY SKILLS
B4. Use tools, equipment, and machine processes safely and correctly in the manufacture of a product; (e.g., appropriate selection of welding process; mig, tig, spot, arc or gas for the task) 

B4.1 use appropriate procedures (e.g., correct machine set-up, operational safety procedures, setting temperature, current for material type and thickness) when setting up, maintaining, using, and storing tools and equipment used in manufacturing and production processes;
B4.2 use the proper procedures to prepare materials for production (e.g., measuring, marking, cutting to rough length, grinding, cleaning, and de-burring, preparing for material welding, relief grooves, cuts to allow appropriate weld penetration, scale removal, preheating process prior to welding);
	C.TECHNOLOGY,THE ENVIRONMENT, AND SOCIETY
OVERALL EXPECTATIONS 
By the end of this course, students will: 
C1. demonstrate an understanding of the importance of using sustainable and environmentally friendly manufacturing practices;
C2. explain the importance of manufacturing to the economy and society.


SPECIFIC EXPECTATIONS
C1. Technology and the Environment

By the end of this course, students will: 
C1.1 demonstrate an understanding and application of the three Rs in a manufacturing facility (e.g., reduction of waste through efficient selection and conversion of materials, reuse of materials when possible, effective collection and recycling of materials and/or fluids);
C1.2 demonstrate the use of proper techniques for the disposal of obsolete and/or waste products; 
C1.3 describe the advantages and disadvantages of using various renewable and sustainable energy sources (e.g., solar, hydrogen fuel cell, wind, geothermal, tidal) and alternative fuels (e.g., biodiesel, ethanol) in manufacturing;
C1.4 follow environmentally responsible practices during the design and manufacture of a product (e.g., minimize waste, consider using renewable or recyclable materials, design and manufacture products that last or can be repaired as opposed to throwaway products, use processes that have minimal impact on workers and the local environment). 
	C2.
	Technology and Society


By the end of this course, students will: 
C2.1 describe the effect the manufacturing industry has on the local community and explain the advantages of manufacturing locally;

C2.2 explain how the globalization of manufacturing industries affects Canadian society locally, provincially, and/or nationally (e.g., explain the effects of trade agreements, worker health and safety standards or the lack of such standards,
PROFESSIONAL PRACTICE AND CAREER OPPORTUNITIES
OVERALL EXPECTATIONS 

By the end of this course, students will: 
	D1. demonstrate an understanding of and compliance with health and safety legislation, standards, and practices, including methods to address deficiencies, as they relate to the manufacturing industry;
D2. demonstrate an understanding of career opportunities in the manufacturing industry and the training and certification required for these careers.


D. PROFESSIONAL PRACTICE AND CAREER OPPORTUNITIES

D1.1 handle materials safely and appropriately in compliance with the information included in the Material Safety Data Sheets (MSDS) from the Workplace Hazardous Materials Information System (WHMIS); (e.g., ensure a well-ventilated and exhaust system is functional when welding, use of a respirator where required, using proper identification labels for discarding of heated materials due to welding heat)
D1.2 Demonstrate an understanding of and compliance with health and safety legislation, standards, and practices related to the manufacturing industry; (e.g., the proper use of ladders, fall arrest systems and protective clothing.)
D1.3 demonstrate the safe use and handling of materials, tools and equipment in compliance with safety manuals, instructions, and institutional requirements; (e.g., employing proper safety procedures such as protection around water, setting proper voltage, wire feed rates, weld beads appropriate to the job being undertaken)

D1.4 use and maintain protective clothing and equipment as required to ensure their own and others’ safety in the work environment; (e.g., including the use of welding protective screens and ventilation systems)
INTERNET ACCEPTABLE USE AGREEMENT FORM

The form below is a sample agreement form that can be used with your board Internet use policy and guidelines.

	INTERNET ACCEPTABLE USE AGREEMENT FORM

	To Students:

I, the undersigned, indicate by my signature that I have read and understand fully the Acceptable Use Policy and related guidelines. I agree that I will abide at all times to the rules and responsibilities as outlined in the Acceptable Use Policy and related guidelines. I also agree that I clearly understand the consequences of my failure to abide by these rules and regulations.

To Parents/Guardians 

As a parent or guardian signing below, I indicate that I understand the rules, regulations and consequences of misuse governing my son or daughter’s use of the Board’s computer and information technology facilities and resources. I understand that all Board staff will make every attempt to ensure proper and acceptable use in line with relevant policies, laws and regulations.  I hereby allow my son or daughter to access the Board’s supervised facilities and resources.

Student Name:

Student Signature:

Date:

Parent/Guardian Full Name:

Parent/Guardian Signature

Date;




To Be Used As An Example Only; Please See Board/School Policy 

MANU Student Conduct Agreement

A signed agreement that outlines the student’s responsibilities is one way of establishing the seriousness of daily safety vigilance. An agreement covers the elements common to all technology classrooms and labs and lays out the framework for a safe and healthy working environment for both staff and students. An example of an agreement is given below.

Safety Awareness

Personal Protective Equipment [PPE]

1. Wear gloves, safety eyewear, aprons, masks, and other PPE as per instructed when using chemicals, heat, biological materials hand or powered instruments and tools.

2. Ensure other workers and customers are protected before performing operations that can be dangerous.

Lift Support and Movement

1. Move heavy objects only with teacher approval.

2. Use assistance to lift items over 20 kilograms (40 pounds) or 2 meters (six feet) in length

3. Secure and support heavy or long objects on approved shelves.

Equipment

1. Operate equipment, chemicals or tools only after receiving proper instruction and permission from the teacher.

2. Never leave equipment, chemicals or tools unattended.

3. Do not attempt to repair any electrical connections, see your instructor.

4. Remove from service any equipment or tools that need repairing.

Storage and Handling of Chemical Substances

1. Understand and follow WHMIS, and MSDS instruction before handling chemical substances.

2. Secure all flammable chemicals and corrosives in approved cabinets.

3. Maintain good housekeeping practices when dealing with chemical substances.

4. Be responsible for cleaning up your workstation, tools and work area.

5. Sort recyclable liquids and solids and biological materials into proper approved storage
         containers
STUDENT CONDUCT AGREEMENT A signed agreement that outlines the student’s responsibilities is one way of establishing the seriousness of daily safety vigilance. 
	STUDENT CONDUCT AGREEMENT FORM

	I, 
 agree to:

Ensure a safe workplace

1. Inform teachers of all injuries, damaged equipment and potentially dangerous situations.

2. Make sure I know all fire exits and power shutdown switches and how to use them during emergency situations.

3. Not compromise the safety of others through horseplay or aggressive action.

4. Only use equipment when properly trained, always with any necessary personal protective equipment, and when I fully understand all related safety issues

5. Ask for assistance from the teacher when I am unsure of the proper procedures or health and safety issues

Prescribed and Non-prescribed Medications

1. Report any use of prescription medications and inform teachers of any possible side effects of the medication [e.g. penicillin, phenobarbital] 

2. Report any use of non-prescription medication and any possible side effects of the medication [e.g. Reactine, Benadril, cough syrups]

3. Never enter a shop or lab carrying, or under the influence of illegal substances

Consequences for Improper Action

I understand that failure to comply with this agreement may result in injury to myself or others, and that failing to comply with safety procedures may result in my temporary or permanent removal from the class or shop.

I have read the above and understand the expectations and consequences.
Student signature:
________________________________________

Parents signature
________________________________________

Date:
________________________________________

Note: No practical work will be done until this form is signed.


	SECTION 2: SAFETY DATA SHEETS


SECTION OVERVIEW

This section contains Safety Data Sheets (listed in alphabetical order) that can be used as:

· Student handouts

· Safety posters (can be mounted in and around specific equipment or bulletin boards)

· Teacher notes in project binders, safety binders or assessment plans

· Information that can support a lesson(s) 

Safety Data Sheets contain information specific to various common tools and procedures. Before using them, ensure they accurately describe your own particular facilities and equipment, and that they align with specific manufacturer’s safety instructions.

NOTE:

All materials within this document are to be considered as suggestions and recommendations only.  These are not legal documents and are not to be considered as legal requirements or as official policy. OCTE or the individual contributors makes no claim to the accuracy or the completeness of the enclosed documents and accepts no responsibility for any damages pertaining to their use. Users of this document should not assume all warnings and precautionary measures are contained herein, that additional information or measures are not required, or that local by-laws, regulations or Board policies are explicitly included.

Please see specific equipment manuals for further safety information, as well as local, Board and school policies and regulations. Please review exemplar TMJ OCTE lab Safety NET resource documents for experienced teacher tips and customization options for your course projects.
	Air Compressor



	1. Wear Personal Protective Equipment (PPE) such as, safety goggles, face shields, gloves hearing protection and proper clothing.

2. Do not operate pneumatic tools without the instructor’s permission.

3. Know the purpose of each tool you use, and use each for the specific task it was designed for.

4. Use only when tools have been demonstrated and 

5. Always use the carrying handle to transport the compressor 
6. Always leave sufficient space (at least 5 meters) between the compressor and the work area in particular when using tools for spraying of liquids

7. The compressor must be placed on a stable surface 

8. Never clean the machine with liquids or solvents when cleaning. Disconnect the machine from the electricity supply by removing the plug and use a damp cloth only.

9. The compressor is designed for air compression only and must never be used for any other type of gas.

10. Never direct the jet of air towards persons or animals or your body.

11. When using compressed air, you must know and comply with the safety precautions to be adopted for the single applications (inflating, pneumatic tools, painting, washing with water based detergents only, etc.)
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Arc Welding



	1. PROTECT YOURSELF by wearing rubber closed toed footwear and leather or flame resistant coats, beanies, coveralls and gauntlet welding gloves.

2. PROTECT YOUR HEAD AND EYES by wearing an approved welding helmet equipped with a minimum shade 10 lens and protective cover glass. Observers must also wear approved EYE PROTECTION.

3. No entry for anyone with metal implants, pace makers or contact lenses.

4. Always place a suitable barrier around the work area to protect others from arc radiation. Use shaded screens (shade 8 minimum) when possible.

5. Be aware of others at all times when welding. Notify others with “Watch Your Eyes” when you are about to weld. 
6. When welding, the area and equipment must be free of water and your gloves and footwear dry.

7. Ensure all connectors are fastened.

8. Ensure the ventilation system is turned on and working.

9. Check for flammable substances before beginning to weld.

10. Always ground to your work piece and be aware of any bearings installed on the work piece.

11. Take breaks to help relieve arm fatigue.

12. Always pick up hot pieces using tongs or pliers.

13. Ensure eye protection is in place when chipping “slag” or grinding a weld.

14. Always ensure the teacher is aware of any equipment problems.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Band Saw (1)



	1. Wear Personal Protective Equipment (PPE) such as safety glasses, safety goggles, face shields and proper clothing as appropriate.

2. No loose clothing, long hair or jewelry is allowed in the shop.

3. Do not operate the band saw without the instructor’s permission.

4. Follow the manufacturer's instructions for changing tool accessories.

5. Be aware of the position of the on/off switches and emergency STOP button.

6. Make all adjustments with the power off.

7. Use both hands and keep fingers at least 10cm (4") from the blade at all times; adjust guard prior to turning the saw on.

8. Keep upper guide less than 5mm (1/4") from the material being cut.

9. Plan your cuts carefully. Saw curves gradually. Sudden twists may cause the blade to bind or break. Use relief cuts if necessary.

10. If the blade breaks, turn the power off immediately and step back. Inform the instructor immediately.

11. Always make short cuts first. Avoid backing out of cuts with the power on. Backing out of a cut may cause the blade to come off of the drive wheel.

12. Do not cut cylindrical stock without the use of a V block clamp.

13. Remove scrap pieces from the table only after the blade has stopped.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Band Saw (2)



	1. Always operate the saw from the front never from the side.

2. Do not leave the band saw until the blade has stopped.

3. Ensure that the blade is running at full speed before starting a cut.

4. Cut on the waste side of your line, leaving the pattern line on the work.

5. Keep your hands beside or behind the blade. Never in front. Use a push stick on small pieces.

6. Make sure all guards are in place and properly adjusted. Ensure all band wheels are enclosed. 

7. Ensure the blade is tracking correctly and runs freely in the upper and lower guide rollers. Ensure the blade is under proper tension. See your instructor for guidance.

8. Use band saw blades that are sharp, properly set and otherwise suitable for the job (e.g., the right tooth pitch; tooth form; blade width). 

9. Hold stock firmly and flat on the table to prevent the stock from turning and drawing your fingers against the blade. 

10. Use a push stick when you remove cut pieces from between the fence and saw blade or when your hands are close to the blade. Keep your hands on either side of the blade - not in line with the cutting line and the blade.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Band Saw - Horizontal



	1. Wear Personal Protective Equipment (PPE) such as safety glasses, safety goggles, face shields, gloves and proper clothing as appropriate.

2. No loose clothing, long hair or jewelry is allowed in the shop.

3. Do not operate the band saw without the instructor’s permission.

4. Be aware of the position of the on/off switches and emergency STOP button.

5. Make all adjustments with the power off.

6. Clamp work firmly into the vice. Long material must be supported.

7. Ensure the blade is tracking correctly and runs freely through the blade guides. Ensure the blade is under correct tension. See your instructor for guidance.

8. Ensure that the blade is running at full speed before beginning a cut. 

9. Allow the upper head assembly to come down slowly until the blade teeth are cutting the material. 

10. Never push down on the cutting head while the saw is cutting.

11. Turn off the machine and bring it to a complete standstill if the blade is to be lifted out of an incomplete or jammed cut.

12. Turn off the machine and bring it to a complete standstill before removing material from the vice area or making adjustments.

13. Ensure the cutting head is locked in the up position before removing work piece from the vice.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Belt and Disk Sander



	1. Wear Personal Protective Equipment (PPE) such as safety glasses, safety goggles, face shields, gloves and proper clothing as appropriate.

2. No loose clothing, long hair or jewelry is allowed in the shop.

3. Do not operate the belt and disc sander without the instructor’s permission.

4. Be aware of the position of the on/off switches and emergency STOP button.

5. Remove all the sawdust around the belt/disc sander. 

6. Do not operate if the abrasive paper is loose or torn. 

7. Ensure that power is off when changing the belt. 

8. Sand only on the rotating-down side surface of the disc-sander and keep your work firmly on the machine table. Do not free hand sand. 

9. Sand only on dry wood. Never sand metal. 

10. Belt sander roll end and side guards should be properly adjusted and in good condition. 

11. Do not apply excessive force toward the belt or disc. Let the machine do the work.

12. When working on small pieces, be careful to keep your fingers and knuckles away from the sanding disk.
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Chemical Handling



	Many operations in manufacturing involve different types of chemicals. Make sure you know how to handle these chemicals: their use, as well as storage and disposal procedures.

1. Before handling any chemicals, ensure you understand the safe handling procedures as outlined on container labels, WHMIS data sheets, designated instructions or posted classroom procedures as appropriate. If you are unsure, see your instructor before proceeding.

2. Place any chemicals in approved, labeled containers ONLY.

3. DO NOT mix chemicals without prior knowledge of the consequences.

4. Discard any used chemicals in approved disposal containers ONLY. Inform your instructor of near-full containers. DO NOT dispose of chemicals down drains. Ask your instructor for proper disposal methods and procedures.

5. Ensure that there is adequate ventilation when using chemical substances.

6. Do not use any chemical for any other purpose other than what it is designed for.

7. Use appropriate PPE (personal protection equipment) at all times when handling chemicals. PPE includes eye protection, skin protection, gloves, aprons or coveralls, foot protection, as required under safe operating procedures.

8. Take note of expiry dates and storage requirements of chemicals. Do not use chemicals beyond their expiration.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	CNC Machine


	1. Any CNC machine may operate automatically without warning. Only a trained individual familiar with the software, machine and computer system should operate this equipment.
2. Be aware of the position of the on/off switches and emergency STOP button.

3. Keep the immediate work area around the CNC machine clear of materials that might cause interference with machine operation.

4. Do not leave the CNC machine unattended when power is on to any electronics.

5. Material cutting operations can create debris. Ensure guards are in place and proper eye protection is worn.

6. CNC machines in motion can create pinch points in normal operation. Be aware of all areas that may be potentially hazardous when the CNC machine is in motion.

7. CNC machines may use hard stops as part of a normal setup. While in motion these stops may be contacted creating a crush point. .

8. Mechanical drives are in use while the CNC machine is in operation. Do not attempt to service, adjust or otherwise touch these components while the machine is on.

9. Certain components of CNC machines are heavy. Use caution when lifting or moving these components. Use team lifting or mechanical lifts when necessary to avoid personal injury.

10. CNC operations such as plasma arc cutting, machining and routing can cause high noise levels that can exceed safe limits. Use hearing protection as necessary to prevent permanent loss of hearing.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR



	CNC Lathe

	1. Any CNC machine may operate automatically without warning. Only a trained individual familiar with the software, machine and computer system should operate this equipment.

2. Be aware and know the position of the on/off switches and emergency STOP button.

3. Keep the immediate work area around the CNC machine clear of materials that might cause interference with machine operation.

4. Do not leave the CNC machine unattended when power is on to any electronics.

5. Material cutting operations can create debris and noise. Ensure guards are in place and proper eye, and hearing protection is worn.

6. Ensure that the work-piece is secure in the chuck before switching on the machine.

7. Ensure all tool bits are sharp and properly secured at the correct height and location before attempting to remove any material from the work-piece.

8. Do not attempt to remove large depth of cut from material at a single pass.

9. When turning a work piece that overhangs 4 inches or more from the chuck, a live center must be used to support the work-piece to avoid it from slipping out of the chuck.
10. Make sure the work-piece size is within the limits of the machine traverse on all axes

11. CNC machines in motion can create pinch points in normal operation. Be aware of all areas that may be potentially hazardous when the CNC machine is in motion.

12. Never attempt to make any adjustments or measurements to the work-piece set up until the CNC Lathe has completely stopped.

13. Move tool holder as far as possible from the chuck while setting up work in the chuck to avoid injury to your hands.

14. Mechanical drives are in use while the CNC machine is in operation. Do not attempt to service, adjust or otherwise touch these components while the machine is on.
15. Certain components of CNC machines are heavy. Use caution when lifting or moving these components. Use team lifting or mechanical lifts when necessary to avoid personal injury.
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	CNC Milling Machine

	1. Any CNC machine may operate automatically without warning. Only a trained individual familiar with the software, machine and computer system should operate this equipment.

2. Be aware and know the position of the on/off switches and emergency STOP button.

3. Keep the immediate work area around the CNC machine clear of materials that might cause interference with machine operation.

4. Do not leave the CNC machine unattended when power is on to any electronics.

5. Material cutting operations can create debris and noise. Ensure guards are in place and proper eye, and hearing protection is worn.

6. Ensure that the work-piece, the work holding device and cutting tool are securely mounted before taking a cut.

7. Make sure the work-piece size is within the limits of the machine traverse on all axes

8. CNC machines in motion can create pinch points in normal operation. Be aware of all areas that may be potentially hazardous when the CNC machine is in motion.

9. Never attempt to make any adjustments or measurements to the work-piece set up until the CNC milling machine has completely stopped.

10. Move table as far as possible from the cutter while setting up work in the vise to avoid injury to your hands.

11. Mechanical drives are in use while the CNC machine is in operation. Do not attempt to service, adjust or otherwise touch these components while the machine is on.

12. Certain components of CNC machines are heavy. Use caution when lifting or moving these components. Use team lifting or mechanical lifts when necessary to avoid personal injury.
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR



	Drill Press



	1. Make sure that your FACE SHIELD AND SAFETY GLASSES are in place before you start the drill press.

2. Keep work area and floor clean and free of oil, grease and debris

3. Always tie back long hair and keep your head and clothes well away from all moving parts of the drill press and never operate with gloves.

4. Operate only after you have received instruction and permission from instructor.

5. Select only drills that are sharp, in good condition and suitable for the job.

6. Remove CHUCK KEYS/WRENCHES from the drill chuck before starting the machine. Never secure CHUCK KEYS/WRENCHES to the column of drill press with a chain.

7. CLAMP THE WORK SECURELY to the table before starting the machine.  Attempting to hold the work under the drill with one hand can result in serious and painful injuries.

8. Operate drills at the proper speed and feed. Forcing or trying to feed too quickly can cause drills to break or splinter with the chance of serious injuries.

9. If work slips from the clamp, never attempt to stop it with your hands. Never reach around or in back of any rotating drill. Use a V-block for round stock.

10. Always ensure that the machine is Locked Out before you attempt to change the belt for speed regulation.

11. If the drill sticks in the work, stop the motor and rotate the drill by hand to free it from the work.

12. As the drill begins to break through the work, ease up on the drilling pressure and allow the drill to break through gradually

13. File or scrape all burrs from drilled holes.  Be sure that the file is fitted with a proper handle.

14. Always clear away chips and curls with a HAND BRUSH only when machine has come to a complete stop – do not use your hands.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR
Radial Arm Drill Press

	1. Make sure that your FACE SHIELD OR SAFETY GLASSES are in place before you start           the drill press.

2. Keep work area and floor clean and free of oil,  grease and debris

3. Always tie back long hair and keep your head and clothes well away from all moving parts of the drill press and never operate with gloves.

4. Operate only after you have received instruction and permission from instructor.

5. Select only drills that are sharp, in good condition and suitable for the job.

6. Remove CHUCK KEYS/WRENCHES from the drill chuck before starting the machine. Never secure Chuck keys/wrenches to the column of drill press with a chain.

7. CLAMP THE WORK SECURELY to the table before starting the machine.  Attempting to hold   the work under the drill with one hand can result in serious and painful injuries

8. Align drill bit to your spot mark on work piece by moving the  radial arm and drill head

9. Clamp the radial arm and drilling head before starting to drill

10. Operate drills at the proper speed and feed. Forcing or trying to feed too quickly can cause drills to break or splinter with the chance of serious injuries.

11. If work slips from the clamp, never attempt to stop it with your hands. Never reach around or in back of any rotating drill. Use a V-block for round stock.

12. Always ensure that the machine has come to a COMPLETE STOP and has been switched off before you attempt to change the belt for speed regulation.

13. If the drill sticks in the work, stop the motor and rotate the drill by hand to free it from the work.

14. File or scrape all burrs from drilled holes.  Be sure that the file is fitted with a proper handle.

15. Always clear away chips and curls with a HAND BRUSH only when machine has come to a complete stop – do not use your hands.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Electrical Hazards



	Touching an exposed electrical wire or electrical equipment that has not been grounded properly causes shocks. Shock can vary from a slight tingle to a rocking jolt. A very severe shock can cause death. Do not touch equipment or electrical wires that have been exposed to fluids.
Protect yourself against shocks by following these rules:

1. Check the condition of electrical cords on equipment. Report all problems to your instructor immediately. Replace worn or damaged cords. 

2. When disconnecting a cord, pull on the plug. Never pull on the cord. You may loosen the wires and get a shock.

3. Never handle electrical equipment with wet hands or while standing in water.

4. Wear rubber-soled shoes to prevent shocks. Rubber does not conduct electricity.

5. Be sure an appliance is turned off before plugging it into an outlet.

6. Make sure you use proper power supplies and cables designated for use with specific pieces of equipment. 

7. Store all electrical equipment in areas designated by your instructor.

8. Never change or interfere with the operating environment set up by someone else without permission.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Facility Emergency Procedures



	1. Make sure you know the location of all fire alarms, emergency exits, and emergency power stop buttons

2. EMERGENCY PROCEDURES AND EVACUATION ROUTES must be clear at all times, and occupants must know and understand these procedures and routes.

Location of Emergency Exits and Fire Alarms:

Locations of Emergency Stops:

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR



	Fall Protection



	Studies of accidents in the service industry show that most injuries are caused by falls. Observing a few simple rules will help to avoid most accidents of this type. 

The points below give guidelines for preventing falls.

1. Walk; do not run. 

2. Keep floor clean and dry. A wet floor is slippery, so wipe up any spills immediately. Sprinkle salt on any spots that are still slippery until the floor can be thoroughly washed. Warn others of slippery conditions.

3. Wear low-heeled comfortable shoes with rubber soles, these grip the floor well. 

4. Keep floor mats flat to prevent stumbling. Wrinkled mats or ones with curled corners can cause falls. 

5. Keep work areas and traffic lanes clear. Electrical cords should not extend across traffic lanes. Put mops and brooms away promptly. Never leave boxes or crates in the aisles. 

6. Look where you are going at all times. Get assistance to carry items that can block your vision.

7. Use a stepladder, never a chair or table, if you need to reach something on a high shelf.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Fire Extinguishers



	1. If you see a fire, call for attention, get everyone out, pull fire alarm.

2. Stay calm. 

3. If using a fire extinguisher:

· PULL THE PIN, AIM LOW AT BASE OF FIRE
· SQUEEZE HANDLE, SWEEP SLOWLY AT BASE OF FIRE
· STAY LOW TO AVOID HEAT AND SMOKE
4. Have the fire department check to make sure the fire is out.

5. Ventilate when fire is completely out.

Learn and know the types of fire extinguishers (see below):

CLASS A

water

[image: image1.png]



Ordinary Combustibles: paper, cloth, wood, rubber, many plastics.
CLASS B

CO2

[image: image2.png]



Flammable Liquids:

oil, grease, gasoline, some paints, solvents etc.
CLASS C

dry chemical

[image: image3.png]



Electrical:

wiring, fuse boxes, electrical equipment etc.
CLASS D

special liquid or powder

[image: image4.png]



Combustible Metals: magnesium, sodium.



	First Aid



	The immediate response to an emergency often involves First Aid. First Aid involves assisting an injured person until professional medical help can be provided. 

The general action tips in the list below should be followed in an emergency. They do not replace the need to be properly trained in first aid. Your teacher will provide you with instructions in what to do in cases of emergencies.

1. Check the scene for dangers, (e.g. electrical shock hazards, chemical spills, hot objects, fire), stay calm and call out for help. Do not touch the victim until immediate dangers such as electrical current are removed.

2. Assist if asked by your teacher to keep the victim comfortable and calm.

3. Call the office for medical help if requested by the teacher.

4. Care for the victim by administering first aid according to your teacher’s instructions.

5. Help keep people who are not needed away from the victim.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	First Aid Kits

	ALL INJURIES MUST BE REPORTED TO MAIN OFFICE

REPORT ANY USE OF FIRST AID KIT TO TEACHER TO ENSURE THAT ANY SUPPLIES THAT ARE USED ARE REPLACED
Suggested list (add items specific to your needs) See WSIB Regulation 1101, Required first aid kit items (at http://www.wsib.on.ca/wsib/wsibsite.nsf/Public/PreventionYHSRR)

DATE CHECKED: 

CHECKED BY: 

ITEM

Number

St. Johns Ambulance First Aid Manual


Masks


Disposable latex gloves


Pair of scissors


Plastic Emesis basin


Wooden splints


Rolls of splint padding

Adhesive strip bandages


3"x3" sterile gauze pads


4" compress bandages


6" Tensor bandages


Triangular bandages


Safety Pins


Sterile gauze bandages


Sterile gauze field dressing


1 ½" width roll adhesive tape


Antiseptic swabs


Burn cream

Instant cold packs


AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Fluid Power Systems

(HYDRAULICS/PNEUMATICS)

	1. Wear safety glasses to protect against flying objects or fluid spray from ruptured lines. Wear appropriate PPE (gloves, apron, etc.) when working with hydraulic fluids.
2. Do not wear loose clothing that could catch in rotating or moving parts. 

3. Do not operate controls in a reckless manner that might cause hydraulic shock and damage.
4. Observe extreme caution when starting up equipment for the first time and after a modification has been made.

5. Never use damaging blows or excessive force when disassembling or assembling fluid power equipment.

6. If the system has an accumulator, be sure its pressure energy is released before it is serviced or modified.
7. Prior to working on a system, lock out any electrical circuits to motors, controls, etc.
8. Shore up or block machine parts that may drop because of gravity.

9. To relieve any residual pressure from a system, bleed fluid by cracking fittings with a rag over the joints until the pressure is zero. 

10. When disassembling a component part, be careful not to unload a spring force that may cause parts to fly.

11. Do not over torque bolts or other fasteners, because doing so may distort housings and affect internal moving parts.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	General Housekeeping



	1. Everything has a proper storage location.

If you don’t know where it is, please ask.

If you do know, put it back.

2. If it is broken, report it.

If it doesn’t work, report it.

If it’s broken or doesn’t work, don’t use it.

3. Dirt, dust, debris are harmful to your safety and health.  Even if you didn’t put it there, pick it up, clean it up, or move it aside.

4. If you spill or drop any fluid on the floor, clean it, or use absorbent materials.  You are responsible for prevention of injuries.

5. Never block fire exits, fire pull alarms, doorways, aisles, and electrical breakers of machine switches for any reason at any time.

6. Chemicals all have proper storage containers.
Make sure you use them.
Never mix chemicals.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Grinder – Portable Angle

	1. WEAR PERSONAL PROTECTIVE EQUIPMENT INCLUDING GLOVES, FACE SHIELD and Safety GLASSES when using a grinder.

2. Check the grinder disk for any flaws before using.

3. Make sure guards are in place and operating properly 

4. Inspect cord for damage before use

5. Always disconnect power when mounting a new disc or attachments.

6. Ensure that the grinder disk is secured and seated properly on the arbor.

7. Check the immediate area for any fire hazards or flammable materials.

8. Make sure you use a portable shield to protect other workers.

9. Start the grinder off the work, grip the grinder solidly with two hands (beware of the torque). Also make sure you have a solid stance before starting to grind.

10. Aim the sparks towards the floor and away from others.

11. When you have finished grinding raise the grinder off the work and allow it to stop on its own.

12. If the grinder is dropped during use it should be thoroughly inspected by the teacher before being used again.

13. Always lay the Grinder down with the Disk facing up and on its Back. 

14. NEVER CUT CLOSED CONTAINERS OR BARRELS
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Grinder – Bench and Pedestal

	WEAR PERSONAL PROTECTIVE EQUIPMENT INCLUDING AN APPROVED SAFETY SHIELD AND SAFETY GLASSES when using a grinder, even if the grinder is equipped with protective glass defection shields. 

Gloves are not be worn when operating grinder.

Keep work area clean and free of oil, grease and debris

Operate only after you have received instruction and passed a proficiency test.

Always check the CLEARANCE OF THE TOOL REST before starting work.  Clearance should never be more than 3mm or 1/8 inch. Lockout grinder when making adjustments.

Always ensure a new grinding wheel has the CORRECT RPM RATING for the grinder it is being installed on.

When mounting or replacing any grinding wheel, always ensure that it fits properly on the shaft.

When installing the grinding wheel to the spindle, be certain the blotters are affixed to both sides of the wheel and that washers and nuts are of the correct size and are tightened securely.

When starting up any grinding wheel, STAND TO ONE SIDE out of line with the wheel and make sure no one is in line with the grinding wheel in case of breakage.

Only grind on the face of the wheel, Use the entire face to avoid grooving the wheel.

Always FEED THE WORK TO THE WHEEL GRADUALLY.  Too much pressure or striking the wheel suddenly may cause it to fracture.

Ensure there is ample water for cooling work piece at the machine

STOP THE GRINDER IMMEDIATELY IF IT BEGINS TO CHATTER OR VIBRATE.  NEVER USE TOOLS OR HANDS TO STOP ANY GRINDER.
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Hand Tools



	Hand tools in poor condition are responsible for a vast number of injuries.

1. Wear EYE PROTECTION whenever using hand tools.

2. Have a proper storage location for your tools to protect them from loss or damage.  After use, clean and RETURN THEM TO THEIR PROPER PLACE so they are always ready when you need them.

3. Never leave tools on floor, hanging over edges, on ramps or hoists where they could be forgotten or cause a tripping hazard.

4. When tools become worn or damaged, they should be repaired or replaced immediately. Show your instructor.

5. Use chisels, knives, blades that are sharp. Do not use blunt tools.

6. Use tools only for their intended purpose. For example, screwdrivers should not be used as pry bars – if they bend under load they are no longer useful and may be dangerous to use as a screwdriver.

7. Files should not be used as pry bars – they are extremely brittle and when breaking will release fragments which could injure or blind you.

8. NEVER STAND BEHIND anyone who is swinging a hammer.  If you have to observe what is being done, stand off to the side out of the way of the hammerhead.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Horizontal Milling Machine 1



	1. Wear Personal Protective Equipment (P.P.E.) such as safety glasses, safety googles, and face shields. No loose clothing, long hair or jewelry is allowed in the shop. 

2. Operate the milling machine only after you have received instruction and permission from instructor.
3. Be aware of the position of the on/off switches and emergency STOP button.
4. Keep work area clean and free of oil, grease and debris.
5. Make sure spindle, beds and control handles are working properly before operating.

6. Ensure work holding device is mounted securely to the table.
7. Ensure work piece and cutter are mounted securely before starting motor and making a cut.

8. To avoid injury when setting up or measuring work, move the machine table as far as possible away from the cutter.

9. Do not attempt to setup or measure a work piece until the cutter is completely stopped. Keep hands away from revolving parts.

10. When setting up or removing a milling cutter from its holder or arbor, ensure proper procedures are followed in order to avoid injury.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Horizontal Milling Machine 2



	1. When setting up a cutter on the horizontal milling machine, always make sure that the cutter is properly keyed to the arbor
2. When assembling or disassembling a tool on the arbor with an over-arm support, always leave the support in place when loosening or tightening the arbor nut.
3. Tighten the arbor nut with a wrench. Never use a hammer to strike the wrench when tightening the nut.
4. Ensure all tool bits are sharp and not damaged. Show your instructor any problems with the tooling.
5. Do not operate the milling machine until you have established proper speeds, stops, and tool depths.

6. Make sure you have proper speeds and feeds for the type of material, tooling and the operation to be performed.
7. Be sure to have firm footing when operating the milling machine.

8. When the machine comes to a complete stop, remove cut off material and chips with a brush. Never use your hands to clear cut materials.

9. Turn milling machine off immediately if it does not sound correct or if there is excessive vibration.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Lifting



	A strain is a feeling of stiffness or soreness from using muscles too long or the wrong way. Strains usually occur in the lower back, the weakest point of the spinal column. In food service, lifting heavy loads incorrectly often causes strains. Once your back has been strained or weakened, it can easily be injured again.

1. You can prevent back strain by lifting with your strong leg muscles. When you must lift a heavy object, squat with knees bent, feet apart, and back straight. With your arms straight, get a firm grip on the load. Stand up keeping your back straight. Make your leg muscles do the work. Do not twist or bend.

2. Set objects down by using the same method in reverse. Ask for help if the object is too heavy. Use a cart to carry heavy objects any distance.

3. Heavy articles should be stored on the bottom shelves.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Metal Cut Off (Chop) Saw



	1. WEAR PERSONAL PROTECTIVE EQUIPMENT INCLUDING GLOVES,FACE SHIELD, SAFETY GLASSES AND HEARING PROTECTION when using

 a metal chop saw.

2. All jewelry must be removed, and long hair tied back securely.

3. Stand to the side of the disc assembly when operating the saw.

4. Students who are left-handed should use their right hand for cutting operations.

5. Clamp material firmly and ensure you are aware of the blade path before you make your cut.

6. Prior to using the saw check the condition of the cord and the abrasive cutting disc.

7. Long stock pieces should be supported safely.

8. Ensure the guard is functioning correctly during operations.

9. When making angle cuts ensure the cutting disc has adequate clearances.

10. Start the saw off the metal and gradually make the cut with even force on the abrasive disc.

11. Pieces of metal that have just been cut will have SHARP EDGES AND WILL BE HOT to touch. 

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Metal Lathe



	1. Wear Personal Protective Equipment (PPE) such as safety glasses, safety goggles, face shields, gloves and proper clothing as appropriate. No loose clothing, long hair or jewelry is allowed in the shop.

2. Keep work area clean and free of oil, grease and debris

3. Do not operate the lathe without proper instruction and the instructor’s permission.

4. Be aware of the position of the on/off switches and emergency STOP button.

5. Make sure headstock, tailstock and tool rests are tight before operating.

6. Ensure your material is secure before starting motor.

7. Ensure all tool bits are sharp and without nicks. Show your instructor any problems with the tooling.

8. Do not operate lathe till you have established proper speeds, stops, tool heights and angles.

9. Make sure you have proper speeds and feeds for the type of material and tool bits, type of operation, and diameter of material. When in doubt, ask.

10. Assume a solid position with your body to the side of the tool. Be sure to have firm footing when operating the lathe.

11. Do not attempt to remove a large depth of cut from material at a single pass.

12. Remove cut off material or chips with a brush after lathe has come to a complete stop Never use your hands.

13. Turn the lathe off immediately if it does not sound right or if there is excessive vibration. 

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Metal Storage & Handling

	1. Storage Rack must have stops on sides so metal won’t fall off.

2. Metal should be stacked flat, by size and type.

3. Metal must be de-burred before returning to rack.

4. Short ends of metal to be placed vertical in a container or in a designated short rack.

Not with the long stock.

5. Ends of stock has to be red flagged if protruding over end of rack.

6. Store plate upright at a 60` angle and secured.

7. Wear gloves when handling metal.

8. Quench ends of metal before returning to rack.

9. When leaving a hot piece of metal to air cool. Mark with soap stone “HOT”

10. Always tap test metal before picking up. It could be “HOT”.

11. Check MSDS before cutting or welding metal.

12. Ask for help when removing or replacing long pieces of stock.

13. Follow all recommended storage and handling procedures.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Metal Storage & Handling

	[image: image5.jpg]



[image: image6.jpg]



AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Oxy-Acetylene Welding (1)



	1. PROTECT YOUR HEAD AND EYES by wearing welding goggles or shield equipped with a minimum shade 5 level of protection. Observers must wear EYE PROTECTION.

2. Cylinders must be secured and upright at all times and stored in a well-ventilated area.

3. Full and empty cylinders must be stored separately. Mark all empty cylinders appropriately.

4. Gas cylinders must have PROTECTIVE CAPS in place for transporting and storing.

5. Ensure that all regulators, hoses, and torches are in good condition, leak-free, and the hoses are equipped with approved FLASHBACK ARRESTORS.

6. Perform leak tests as part of a preventive maintenance procedure.

7. Use only approved pressure-reducing regulators with each gas cylinder.

8. OXYGEN COMBINES WITH OIL AND GREASE to cause violent fires.  Do not use oxygen to blow dust off clothing.

9. Keep equipment free of oil or grease

10. Make certain a fire extinguisher is readily available.

11. PROTECT YOUR SKIN by wearing safety footwear and leather or flame resistant canvas coat and gloves.

12. Do not carry a Butane lighter or other flammables in your pockets

13. Only use a proper striker to ignite torches

14. Check for flammable substances in the vicinity before beginning to weld.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Oxy-Acetylene Welding (2)



	START UP

1. Open oxygen cylinder valve slowly ½ turn to prevent damage to regulator. Then open all the way.

2. Open acetylene cylinder valve 3/4 turn only.

3. Adjust hose pressures using the T-handles on the regulator gauges. 

4. Purge acetylene and the oxygen lines individually before lighting torch.

5. Open the acetylene torch valve 1/3 turn and light the acetylene gas using a striker (before opening the oxygen torch valve).

6. Open the oxygen valve slowly until a neutral flame is achieved.

SHUTDOWN
1. Close the torch acetylene gas valve to extinguish the flame. (A before O).
2. Close the torch oxygen valve.

3. Close the acetylene gas cylinder valve.

4. Close the oxygen cylinder valve.

5. Drain the acetylene gas line by opening the torch acetylene gas valve.  Release the acetylene gas regulator knob (T-handle) on the gauge. 

6. Close the torch acetylene gas valve.

7. Drain the oxygen lines by opening the torch oxygen valve.  Release oxygen regulator knob (T-handle) on the gauge.

8. Close the torch oxygen valve. Oxygen and acetylene gauges for both tank and hose pressures should read zero.

9. Neatly wrap hoses on holder, and place torch handle and tip in a position that will prevent damage to them.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR

	Plasma Arc Cutter



	1. Wear protective clothing when using the plasma arc cutter. Clothing should be wool or cotton, long sleeves, leather shoes (high top), gauntlet gloves and leather apron. 

2. Wear industrial quality eye protection minimum #5 shaded lens for plasma arc cutting operations. 

3. Ensure that the work area is well ventilated when using the plasma arc cutter.
4. The operator should position himself/herself so there will be minimum exposure to fumes during the cutting process.

5. Never use the plasma arc cutter in areas where combustible or explosive gases or materials are located.

6. Never touch any parts on the plasma arc cutter that are electrically connected.

7. Disconnect the electrical power before performing any service or repair on the plasma arc cutter.

8. Never use the plasma arc cutter to cut into an enclosed container.
9. Hearing protection should be worn when using the plasma arc cutter.

10. Use pliers or clamps to hold hot metal. 

11. Quench hot metal or allow it to cool before handling.

12. Make sure others in the work area are protected from plasma arc rays, fumes and splatter
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Power Tools (1)



	1. Wear Personal Protective Equipment (PPE) such as safety glasses, safety goggles, face shields, gloves and proper clothing as appropriate.

2. Do not operate power tools without the instructor’s permission.

3. Do not wear loose clothing or jewelry when operating power tools. Long hair must be tied back.

4. Follow the manufacturer's instructions for lubricating and changing tool accessories. 

5. Keep guards in place and follow lockout/tag-out procedures. 

6. Know the purpose of each tool you use, and use each for the specific task it was designed to do.

7. Always use two hands on the tool when operating. Clamp work piece to a solid surface; do not attempt to hold work piece with hand or foot.

8. Unless it's designed for it, never use a portable electric tool where there are flammable vapors or gases present. 

9. Electrical cords must be in good condition; report any broken, damaged or bare cords. Keep cords away from heat, oil, and sharp edges

10. All power tools must be effectively grounded and/or be of the double insulated type.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Power Tools (2)



	1. If the tool is equipped with a three-prong plug, it should be plugged into a three-holed electrical receptacle. Never remove the third prong.

2. Never use power tools in damp or wet locations or if the worker is perspiring. Moisture helps electricity flows more easily through the body. 

3. Rubber gloves and footwear are recommended when working outdoors in damp conditions.

4. Never carry a tool by its cord, or pull the cord to disconnect it from a receptacle. Never carry a plug-in tool with your finger on the switch. 

5. Unplug tools before replacing any broken, dull or damaged bits or blades.

6. Be careful not to overreach. Keep your balance and proper footing when working with power tools.

7. When you have completed an operation with a power tool, switch it off and lay the tool down in a safe manner after it stops.  Keep the rotating blade or bit away from your legs and body.

8. Keep floor around work area clean.

9. Be sure the power switch for a portable tool is “off” before plugging it in. 

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Programmable Logic Controllers (Wiring Precautions)



	1. Do not attempt to take any unit apart while power is being supplied to the PLC.

2. Do not touch any of the terminals or terminal blocks while the power is being supplied to the PLC.

3. Completely turn off external power when installing or replacing wiring.

4. When wiring in the PLC, be sure that it is done correctly by checking the products rated voltage and the terminal layout.

5. Tighten terminal screws with the specified torque. Loose screws may result in burning.

6. Wiring bare wires to directly to PLC terminals may result in fires. Use solder less terminals or solid wires for wiring.

7.  Do not pull on cables or wires running to the PLC. Bending wires or cables beyond their natural limit may also cause damage.

8. Before turning on the power supply or operating the module after installation or wiring work, ensure that the modules terminal covers are correctly attached.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Robotics



	1. Make sure the robot base is secure to the work surface.

2. Make sure the robot arm has sufficient space in which to move freely.

3. Do not place your hands or fingers or any object within the robot arm’s operating range, particularly while it is in motion.

4. Ensure the device is powered off before approaching or handling the robot.

5. Do not exert force on any part of the system. 

6. Do not use physical or mechanical means to stop the movement of any part of the robot arm.

7. Do not drive the arm into another object or an obstacle.

8. Do not overload the arm. Check the manufacturer’s instructions for maximum workload. The gripper should grasp the workload at its center of gravity.

9. Never leave the arm or gripper under mechanical strain for extended periods of time.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Sheet Metal Shear/Roller/Bender



	1. The floor around the machine should be clean and free of scraps, oil or grease. A suitable non-skid material should be applied to the floor at the work station.

2. Do not operate the machine when other persons are in contact with the machine or the work piece.

3. Obtain assistance for moving large sheets of material to and from sheet metal machines.

4. Keep fingers clear of the area in front and rear of the shear blades.

5. Keep clear of the slip roll nip points.

6. Keep fingers clear of the brake when clamping/forming materials.

7. Be aware of the maximum capacity of the machine. Exceeding the capacity of the machine may be hazardous to the operator.

8.  Never use excessive force when operating sheet metal equipment.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Surface Grinder

	1. Wear Personal Protective Equipment (P.P.E) such as safety glasses, safety goggles, face shields, proper clothing as appropriate. No loose clothing, long hair, or jewelry is allowed in the shop

2.  Operate grinder only after you have received instruction and permission from instructor 

3. Be aware of the positions of the on/off switches and emergency STOP button

4. Keep work area clean and free of oil, grease and debris.

5. Always ring test the grinding wheel before mounting it to the machine.

6. When starting a grinder, always stand to one side of the wheel and make sure no one is in line with the grinding wheel in case of breakage.

7. See that the grinding wheel clears the work before starting a grinder

8. Allow a new wheel to run for about one minute before using it.

9. Never run a grinding wheel faster than the recommended speed on its blotter.

10.  Ensure the wheel guard is in place and it covers ½ of the wheel.
11. Keep the magnetic chuck clean and free of burrs.

12. Always dress the wheel when turning it on.

13. Remove all burrs from the work piece before placing on magnetic chuck

14. Ensure the magnetic chuck is on and holding the work piece by trying to remove the work.
15. Never attempt to clean the magnetic chuck or mount and remove work until the wheel has stopped completely.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	Vertical Milling Machine



	1. Wear Personal Protective Equipment (P.P.E) such as safety glasses, safety goggles, face shields, proper clothing as appropriate. No loose clothing, long hair, or jewelry is allowed in the shop.

2. Be aware of the position of the on/off switches and emergency STOP button.

3. Keep work area clean and free of oil, grease and debris.

4. Make sure spindle, beds and control handles are working properly before operating.

5. Ensure work holding device is mounted securely to the table.

6. Ensure work piece and cutter are mounted securely before starting motor and making a cut.

7. To avoid injury, move the machine table as far as possible away from the cutter when setting up or measuring work.

8. Do not attempt to setup or measure work piece until the cutter is completely stopped. Keep hands away from moving parts.

9. When setting up or removing a milling cutter from its holder, proper precautions should be taken to avoid injury.

10. Ensure all tool bits are sharp and undamaged. Show your instructor any problems with tooling before operating the milling machine.

13. Make sure you have proper speeds and feeds for the type of material being machined. When in doubt, ask the instructor.

14. When the machine comes to a complete stop, remove cut material and chips with a brush.

15. Turn milling machine off immediately if it does not sound correct or there is excessive vibration.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Vise

	1. Vise is to be permanently fixed to tables.

2. When not in use keep jaws closed and handle vertical.

3. Never use a lever or hammer to apply extra force to the vise handle.
4. Support long end of work with a stand.
5. Keep cut side of work close to the jaws.
6. When tightening and loosening vise keep knuckles on the outside of handle.

7. When hammering or bending metal direct force to fixed part of Vise.

8. Use short tugs on handle to tighten or loosen work.

9. Hold material with one hand to ensure it doesn’t drop on floor.

10. Keep hand and fingers clear of the vise jaws before closing them.
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	                          Vise

	

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	Welding Operational Safety

	1. Students are to be certified on all equipment before using, (Certification done by written and practical testing.)

2. Any adjustments to the welding machine settings are to be done while the machine is not under load. 

3. When welding on equipment, double ground the work so the current does not go through bushings or bearings.

4. Never weld in confined spaces unless adequate ventilation is supplied and the air quality is tested and monitored.

5. Never weld or cut on containers that has housed contaminates, or flammable material.

6.  A “Hot “permit must be filled out when cutting or welding in a potential fire hazardous areas.

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




[image: image8.jpg]HOT WORK
PERMIT

The supervisor, in issuing this permit, certifies that all safety factors
have been considered and cared for satisfactorily.

Return this permit upon completion of the job which it is to cover to
the authorizing supervisor. The supervisor will write “complete”, date and
initial across the face of the permit.

AREA OF HOT WORK:

WORKTO BE DONE:

Read the Hot Work Permit Procedure

Work area and equipment has been made
free of flammable, combustible, and
hazardous materials.

Gas Test taken.

guisher on the job?
Smoke alarms covered?
Lines disconnected and/or blanked?
Is a fire watch provided?

Adjoining equipment and operations
considered ok from standpoint of possible
effect on the job.

Other necessary precautions
SPECIFY

APPROVAL
I have personally checked the conditions necessary and as
specified | authorize this “Hot” work to begin.

APPROVED BY. DATE. TIME.

HOT WORK PERMIT IS GOOD FOR HOURS ONLY
THIS PERMIT CAN BE ISSUED FOR ONLY ONE SHIFT. IT BECOMES VOID AT
THE END OF WORK SHIFT DAY.





	Welding Student Safety

	1. Closed toed and Rubber soled shoes with socks must be worn (no slippers or loafers).

Leather ankle supported recommended.

2. Long legged pants must be worn, wool, cotton or denim (No polyester).

3. No Jackets, Hoody’s or garments that implead movement or present a hazard.

4. No jewelry, plastic or metal (rings, bracelets, chains, watches or facial piercings).

5. Hair must be tied back and covered by a beanie.

6. No contact lens to be worn.

7. No entry for anyone with metal implants or pace makers without a doctor’s note. (Magnetic field could cause interference).

8. Safety glasses with a Z87 logo must be worn all the time including over prescription

      eye wear.

9. A Face shield must be worn over safety glasses while preforming operations e.g., drilling, grinding or using a spot welder. 

10. Do not bring any lighters, matches or flammable items.

11. Wear all prescribed safety P.P.E. (personnel protection equipment).

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR




	WHMIS Regulations



	· The acronym WHMIS stands for Workplace Hazardous Materials Information System
· This regulation was introduced into the Occupational Health and Safety Act of Ontario in 1987

· WHMIS gives a worker the right to know what they are handling

· Federal WHMIS applies to importers, manufactures, and suppliers of hazardous materials

· The Hazardous Products Act for controlled products states that national suppliers are to provide labels and Material Safety Data Sheets to all buyers

· MSDS stands for Material Safety Data Sheets
· MSDS is a print out on paper that identifies how to handle, store, use, health effects if exposed, emergency procedures, and protective measures

· MSDS must be updated by the supplier every 3 years
· Under the Occupational Health and Safety Act in Ontario, employers are to make sure that all hazardous materials are labeled appropriately and that a MSD sheet accompanies the product

· On any controlled hazardous product two labels must be properly labeled with a supplier and a workplace sticker

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR

WHMIS Labels

	Supplier labels must be attached to the controlled product container which has detailed information about the product.  Legislation states that 10 kg or more of a controlled product or hazardous material from a supplier must contain the following information:

· both official languages

· have a WMHIS border

· identifies the material or product name (i.e. common name, chemical name, trade name, generic name, brand name, code name or number)

· name and address of the supplier

· reference to a MSD sheet

· WHMIS hazard symbols

In addition to this and if the container has more than 100 milliliters the following information must be on the label:
· risk time factors

· precautionary measures while using or being exposed to the product/chemical

· First aid measures to address immediate injuries and not progressive illnesses

Workplace labels must be identified on a container that is not from the supplier, and must contain the following information: 

· material identifier or product name

· reference to a MSD sheet

· precautionary steps 

· first aid measures
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	MSDS Safety Labels



	GENERIC MATERIAL SAFETY DATA SHEETS FOR PERSONAL ENHANCEMENT PRODUCTS PROTECTED BY TRADE SECRET LAWS (MSDS) 
MATERIAL IDENTIFICATION

_____________________________________________________________________

TRADE NAME/MATERIAL NAME                                                  PRODUCT USE

______________________________________________________________________

OTHER NAMES:

______________________________________________________________________

MANUFACTURER'S/SUPPLIER'S NAME:

______________________________________________________________________

ADDRESS:

______________________________________________________________________

EMERGENCY TELEPHONE:

______________________________________________________________________

FIRST AID PROCEDURE

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	SECTION 3: SAFETY ASSIGNMENTS AND TESTS


SECTION OVERVIEW

This section contains sample tests and assignments related to safety. They are designed as samples that can be used as written or edited for your purposes. They can be used for evaluation of the safety expectations of the course, or as tools to assess the student’s knowledge and understanding of safety. It is recommended that all teachers keep a record of all test or assignment results and/or passports (next section) as verification of each student’s understanding of safe concepts and practices.

The equipment and safety practices in individual facilities will determine how a teacher can best use these resources in the teaching of safe work practices. As well, with the SafetyNET resources online at OCTElab, there are additional resources always being updated, and available for download in .zip files.

NOTE:

All materials within this document are to be considered as suggestions and recommendations only.  These are not legal documents and are not to be considered as legal requirements or as official policy. OCTE or the individual contributors makes no claim to the accuracy or the completeness of the enclosed documents and accepts no responsibility for any damages pertaining to their use. Users of this document should not assume all warnings and precautionary measures are contained herein, that additional information or measures are not required, or that local by-laws, regulations or Board policies are explicitly included.

Please see specific equipment manuals for further safety information, as well as local, Board and school policies and regulations. Please review exemplar TMJ OCTElab SafetyNET resource documents for experienced teacher tips and customization options for your course projects.

Safety Assignment # 1 – Room Inventory and Safety Identification

Use a ruler/straight edge to draw a neat floor plan of your shop and identify the location of the following. Show the work zones around major equipment. Check off each item to ensure you have covered everything:

	Entrance/exit doors
	

	Safety exit
	

	Fire extinguishers
	

	Fire alarm
	

	First aid kit
	

	Power shut-off or emergency “stop” buttons
	

	Electrical outlets
	

	Exhaust fans/hoods
	

	Sink areas
	

	Waste disposal containers
	

	Work tables/work surfaces
	

	Computer work areas
	

	Equipment and tool storage areas
	

	Chemicals storage
	

	Consumable supplies storage
	

	Gloves storage
	

	Apron/coverall storage
	

	Safety glasses storage
	

	Welding Gas storage
	

	Traffic areas
	

	Machining areas
	

	Welding areas
	

	Materials Supplies Storage
	

	
	


Safety Assignment # 2 – General Safety

In groups of two, analyze the issue you have been assigned and provide a detailed description of the safety requirements for that issue. Information for research may be found in a variety of places including textbooks, the Internet, equipment manuals, or from local suppliers. A 5 -10 minute group presentation will be made to the class in which your group will describe the topic and the importance of safety in a manufacturing technology environment. 

· Group 1
Arc/MIG/TIG welding

· Group 2
Oxy-acetylene welding and cutting

· Group 3
Plasma Arc cutting 

· Group 4
Sheet Metal fabrication

· Group 5
Grinding

· Group 6
Engine lathe operations

· Group 7
Drilling operations

· Group 8
Milling operation

· Group 9
Sanding/Finishing

· Group 10
Chemicals, solvents, and fluids

Safety Assignment # 3 – Perform a Safety Audit
Once a month, a group of you will be assigned to perform a safety audit of the workshop. To accomplish this task, the group must first design a safety checklist that will be used for the inspection.  The checklist must include the headings of:

1. First aid kit content status

2. Status of safety equipment

3. Status of fire protection equipment

4. Status of cleaning supplies and equipment

5. Status of storage areas

6. Status of tools and equipment

7. Status of materials storage

8. Status of housekeeping

Your teacher will give you information about safety standards. Prepare a checklist for a safety audit of the shop. When you have approval for your checklist, perform the initial audit and report back to your teacher.

WELDING TEST QUESTIONS
(See Answer Sheets after the test listings)

WELDING HAZARDS TEST

1. What are the ten major hazards in welding operations?

2. What four types of radiant energy should the welder guard against?

3. What effects can metal fumes have?

4. How can sparks and slag be a hazard?

5. What two main hazards may result from the use of electricity?

6. What chemical sources can the welder be exposed to?

7. What are four welding heat sources that may cause fires?

8. What four causes of explosions must the welder guard against?

WELDING SAFETY TEST

1. Name the three natural clothing materials that are recommended for welders.

2. What laundry product reduces the ability of clothing to repel sparks?

3. Why are synthetic materials not recommended for welders?

4. Give two requirements regarding the fit of welding goggles.

5. What device should be used to prevent fumes from accumulating in a poorly ventilated area?

6. What are two functions of earmuffs and earplugs in welding?

7. Name three things that can affect the seal of a full-face respirator.

WELDING FIRES AND EXPLOSIONS TEST

1. How do air ducts and shafts increase fire hazards?

2. What danger is posed by excessive oxygen in the atmosphere?

3. What must flammable gases, vapours, liquids or dusts be mixed with to create an explosive atmosphere?

4. Why should gas cylinders, hoses and torches be kept outside a confined space where welding is being done?

WELDING FUMES AND GASES TEST

1. To reduce the danger of fumes, what is the first, simplest, and most basic precaution?

2. What product, used to treat metal surfaces, will produce phosphine gas?

3. What gas may be produced by degreasing fluids?

4. What plastic material should be stripped or removed from the weld area before welding or cutting?

5. What vapours change into a dangerous gas when acted upon by ultraviolet light?

6. What causes negative pressure in some respirator face pieces?

7. Which is best, a negative or a positive pressure face piece?

8. If the cartridge or canister of an air-purifying respirator becomes hot, what does this signify?

9. Name four factors, other than clothing, that can interfere with the fit of a respirator face piece.

CONTAINERS AND WELDING TEST

1. If it is not known what substance a container has held, what should you assume?

2. Name four substances whose dusts are flammable.

3. What gas may be produced when an acid reacts with a metal container?

4. Name three methods of cleaning a container.

5. Name the five major steps that must be taken before welding or cutting on a container.

6. How does purging a vessel differ from cleaning?

7. What does it mean to "isolate" a container?

8. At the same voltages, why is ac more dangerous than dc?

ANSWERS TO THE WELDING HAZARDS TEST

1. Ten welding hazards:

a. radiant energy

b. temperature extremes

c. fumes and gases

d. noise

e. sparks and slag

f. electrical shock

g. chemicals

h. fires

i. explosions

j. tripping hazards

2. Four types of radiant energy: 

a. visible light

b. ultraviolet light

c. infrared rays

d. x-rays and gamma rays

3. Metal fumes can damage the lining of the lungs. Prolonged exposure can damage other organs, or cause death.

4. Sparks and slag can cause fires, burn exposed parts of the body, damage hearing, and cause eye injuries.

5. Two electrical hazards are: shock and burns.

6. Among the chemical sources the welder may be exposed to are fluxes, anti-spatter compounds, glues, rust inhibitors, paints, degreasers, and plastics.

7. Some welding heat sources which cause fires: 

a. the torch flame;

b. sparks from welding, cutting and grinding; 

c. used welding rods and electrode stubs; 

d. the weld deposit;

e. the base metal; 

f. slag.

8. Some causes of explosions in welding operations: 

a. using equipment incorrectly;

b. using damaged equipment or containers; 

c. welding on a pressurized container;

d. welding in the presence of explosive materials.

ANSWERS TO WELDING SAFETY TEST

1. Cotton, wool and leather are recommended for clothes.

2. Fabric softeners decrease clothes ability to repel sparks.

3. Synthetics may melt where sparks land. Note: some specialized synthetic materials can provide a short-term flame barrier.

4. Welding goggles should fit over safety glasses or spectacles, and should make full-face contact.

5. A portable exhaust fan helps vent an area.

6. Earmuffs and plugs help to exclude noise and sparks.

7. Facial hair, clothing, goggles and spectacles can affect the seal.

ANSWERS TO WELDING FIRES AND EXPLOSIONS HAZARDS TEST

1. Air ducts and shafts carry sparks to distant areas.

2. Excess oxygen causes materials to burn, which are not normally combustible, or causes them to burn more fiercely.

3. Oxygen, (or the oxygen in air) must mix with substances to make them combustible.

4. Because they may leak, allow gases to accumulate in the atmosphere.

ANSWERS TO WELDING FUMES AND GASES TEST

1. Staying to one side of the welding plume is the first defense.

2. Rust inhibitors produce phosphine gas.

3. Phosgene may be produced by degreasers.

4. Teflon-type plastics should be stripped or removed from the weld area.

5. The vapours of chlorinated hydrocarbon degreasers become a dangerous gas.

6. The wearer inhaling causes negative pressure.

7. A positive pressure face piece is best.

8. That a high concentration of gas or vapour is being encountered when a cartridge or 45 canister becomes hot.

9. Sideburns, beard, moustache, eyeglasses and goggles interfere with a respirator's fit.

ANSWERS TO CONTAINERS AND WELDING TEST

1. Assume that the container has held a flammable substance if you are unsure about the contents.

2. Sugar, wheat, coal and wood have flammable dusts.

3. Hydrogen is produced.

4. Three ways to clean a container: 

a. wash with water

b. low pressure steam

c. chemical plus low pressure steam

5. Prior to welding or cutting:

a. identify contents

b. determine internal combustion 

c. clean

d. purge 

e. isolate

6. Cleaning is intended to remove the bulk of liquids, solids and vapours. Purging is intended to displace any remaining vapours and to exclude air.

7. To isolate a container is to disconnect or blank off any pipes leading into it which carry dangerous substances.

8. Alternating current can cause spasms that tighten the grip on a conductor; DC has the power to throw a person away from the conductor.

[image: image9]
Each piece of equipment we use in the manufacturing facility has specific safety and operating guidelines and procedures.  The purpose of this activity is to research the equipment and produce safety information sheets on all of the equipment. The safety sheets will be laminated and used as reference material for all manufacturing technology students. You must include the following information on all safety information sheets:

1. name of equipment   (i.e. Plasma arc cutter)

2. function of equipment   (i.e. Cut and shape metal)

3. safety guidelines

4. correct operating procedures 

You are developing important mandatory material that will be used by all manufacturing students. The information sheets must be precise and easy to follow. Material reference information such as equipment manuals provided by the manufacturer is your main source.  The web may give you additional information as well as the training you received through the manufacturing program. If you have any questions at any time, please ask!

Equipment list:





Due Date:

Engine Lathe






Tuesday March 10th 

Vertical Milling Machine




Tuesday March 24th                                                            Oxy-Acetylene Welder




Tuesday March 31st  

MIG Welder






Tuesday April 7th                          

TIG Welder






Wednesday April 15th  

Horizontal Bandsaw





Tuesday April 21st  

Vertical Bandsaw





Tuesday April 28th   

Arbor Press





            Tuesday May 5th 

Plasma Arc Cutter





Tuesday May 12th    

Squaring Shear





Tuesday May 19th  

Pedestal Grinder





Tuesday May 26th   

Tube Bender






Tuesday June 2nd 

Angle Grinder






Tuesday June 9th  

Robot







Tuesday June 16th

Sample

Manufacturing Facilities Health and Safety Inspection Checklist

Teacher Inspecting: _________________________________

Student Inspecting: __________________________________

Date of Inspection: __________________________________
	AREA INSPECTED
	CONDITION
	ACTIONS NEEDED
	DATE RECTIFIED

	Outlets
	
	
	

	Electrical Equipment and extension cords
	
	
	

	Emergency Power Switches/Breakers
	
	
	

	Fire Extinguishers
	
	
	

	Exit and Light Fixtures
	
	
	

	Exhaust Fans and Hoods
	
	
	

	Traffic Areas
	
	
	

	Ceiling Tiles and Fixtures
	
	
	

	Floor Tiles/Carpet and Surfaces
	
	
	

	Sink Area and cleaning areas
	
	
	

	Chemical Storage and Labeling
	
	
	

	Eye Wash Station
	
	
	

	First Aid Kit
	
	
	

	Safety glasses/goggles
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


Sample WHMIS and MSDS Quiz 

Section 1

Define WMHIS and MSDS.

What is the responsibility of the employer in regards to WHMIS according to the Occupational Health and Safety Act of Ontario?

Section 2: Multiple Choice

1.  If a hazardous material has more than 100 milliliters in one container, the label must have additional information which includes:


a) the companies chemist
b) risk time factor


c) b and d


d) precautionary measures while exposed to the product

2.  Workplace labels must contain a material identifier or product name, reference to a MSDS, precautionary steps, and:


a) an emergency phone number
b) the hospitals phone number


c) first aid measures


d) the company’s phone number

3.  In Canada a suppliers WHMIS label must be written in:


a) French


b) English


b) Chinese


d) both Official Languages

4.  A supplier when selling a hazardous material product must include:


a) a rebate


b) MSDS


b) WHMIS


d) OH&S

5.  A Material Safety Data Sheet should be:


a) kept on file forever


b) read and then thrown out


c) photo copied for all workers
d) placed in a binder and kept for 3 years

Answer Key:
Section 1

1. Workplace Hazardous Material Information System, Material Safety Data Sheets

2. To inform employees of hazardous materials.

Section 2: Multiple Choice: 1. c  2. c  3. d  4. b  5. d

	GENERAL SAFETY QUIZ



	Name:_______________________________

Date:________________



	The following are True or False questions. Write T for true or F for False for each question.

1. If you are uncertain about something in the shop, it is okay to ask a peer.

2. If the fire alarm sounds, continue working until told to stop.

3. Shop equipment needs to be cleaned only at the end of the period each day. 

4. It is okay to bring a drink into the shop as long as none of the equipment is running 

5. Any adjustments to a machine must be made with the power off.

6. It is okay to talk to a person while they are using a piece of equipment, as long as you do not distract them.

7. It is okay to use a flat screwdriver to scrape some old paint off of a piece of metal.

8. The first aid kit can be stored in the school main office so no one steals the contents.

9. A class “D” fire extinguisher is a must in a manufacturing shop.

10. Once you have passed the safety test/quiz on a piece of equipment you may use the equipment any time without permission

11. Minor injuries need not be reported.

12. If a machine does not work, report it to the instructor.

13. All guards must be in place and properly working before using the equipment

14.  A safety zone is an area where shop rules do not apply.

15. As long as no one is using the equipment after you, leave it running until you need it again.

16. Safety equipment is necessary only when power is on.

17. A safety zone is an area where shop rules do not apply.
18. Long hair must be tied back before using any power tool.

19. Before working in a construction shop you should know where the emergency exits are.

20. Any adjustments to a machine must be made with the power off.




	Answer Key

1. F
2. T

3. F

4. F

5. F

6. F
7. F

8. F

9. F

10. F

11. T
12. T

13. F

14. T

15. F

16. F
17. F

18. T

19. T

20. T


	GENERAL SAFETY RULES QUIZ



	Name:_______________________________

Date:________________



	Write the rule that relates to the following statements:

1. Using the proper tool for the job.

2. How to carry tools.

3. Electrical cords and plugs.

4. Portable tools when not in use.

5. Securing your work.

6. Safety guards.

7. Moving parts on equipment.

8. How many persons operate machines at one time, what do partners do?

9. Make sure it is completely stopped before leaving.

10. Broken or damaged tools.




	BAND SAW SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. When using the band saw, _____​​​_____________are required when making curved cuts.

2. You should adjust the _____________ above the work before beginning to cut.

3. Narrow blades are best suited for cutting ________________.

4. When cutting with the band saw, the blade should cut on the ___________ side of the work piece.

5. When using the band saw, plan your cuts carefully. Saw curves gradually. Sudden twists will cause the blade to ______________ or _______________. 
6. When using the band saw, keep your hands ____________ or _____________ the blade. Never in front.
7. Inspect all ____________________ before using any saw.

8. Always support ______________________ pieces.

9. Use _______________________ sticks on small pieces.

10. Use the ______________________ when changing blades.



	WORD BANK: 

relief cuts, upper guide, tight, waste, bind, break, beside, behind, guards, round, push, lock-out




	HORIZONTAL BANDSAW

SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always wear _____________________ when operating the horizontal bandsaw.

2. Ensure all ______________________are fitted, secure and functional.

3. Ensure the hydraulic ______________________ mechanism is functioning and the head of the unit is locked in the upward position...

4. Check the ________________ to ensure it is in good condition...

5. Check for proper blade speed, ________________ and tracking.

6. Clamp the workpiece firmly into the _____________________...

7. Long material must be properly _____________________.

8. Allow the blade to come down__________________ until the teeth are contacting the material.

9. Keep your ______________ away from blade and cutting area.

10. Turn off the machine and bring it to a complete _____________________ if the blade needs to be lifted out of an incomplete or jammed cut.

11. Ensure the cutting head is locked in the ____________________ position before removing the workpiece from the vice.



	WORD BANK:

Safety glasses, upward, supported, damping,  fingers, vice,  guards, slowly, blade, tension, stop




	BELT AND DISK SANDER

SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Wear _______________ goggles or a face shield.

2. Secure the_________________ position before beginning sanding procedures.

3. Check the _________________ of the disk. Work on the downward side of the motion.

4. Wear a _______________ mask when sanding treated wood .Put the vacuum on.

5. Only one person _______________ the machine at a time.

6. Remove _________________ and tie back ______________ hair.

7. Let the work _____________ on the table and do not force it into the disk or the belt.

8. Keep your ______________ away from the edge that contacts the sandpaper.



	WORD BANK:

safety, jewellery, long, dust, table, rest, fingers, rotation, operates




	DRILL PRESS SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always keep _______________ around drill press clean and free of oil and grease.

2. Always wear _________________ when operating the drill press.

3. Keep your _________________ and loose ______________ away from revolving parts.

4. The work piece should be _______________ before drilling to prevent any accidents.

5. Never clear metal chips with your _____________ always use a _____________.

6. Forcing a drill into the work can cause the drill to___________________.

7. Never attempt to hold the work piece by   _____________ when drilling.

8. Always remove _________________________ from a hole that has been drilled.

9. Never leave the ________________   ________________ in the spindle.



	WORD BANK:

Break, floor, chuck, hand, safety glasses, burrs, clamped, clothing, hands, brush, key, head




	FLUID POWER SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always wear appropriate _______________when working with fluid power systems. This includes eye protection (safety glasses) but may also include gloves and aprons to protect against the corrosive effects of some fluids.

2. Before working on a fluid power system, remove _________________ from the system by operating components and/or removing the fluid power source.

3. Some hydraulic fluid vapours can be quite harmful if _____________________. Be aware of this hazard and wear a ventilator if appropriate.

4. Accumulators often contain hydraulic and _________________ pressure. Ensure all pressure in an accumulator is released before working on the device.

5. Be careful not to unload a _____________ _____________ that may cause parts to fly during component disassembly.

6. Over torque of fasteners may ____________________ fluid power components and cause failure of parts when placed under pressure.

7. Use extreme caution when starting up a system for the ________________ time or after having completed maintenance/modifications.

8. Keep your work are clean and free of spilled _________________.

9. Ensure proper ___________________ are in place where there are moving parts of a fluid power system.



	WORD BANK:

distort, pressure, inhaled, fluids, PPE,  spring force, air, first, guards


	PEDESTALGRINDER SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always use a _________________________ or goggles when grinding.

2. Inspect the ___________________ wheel before starting the machine.

3. Check the ___________________ of the tool rest. It should not exceed ______of an inch.

4. When starting up the grinder always ________________ to one side, not directly in front of the wheel.    

5. Check the ________________ of wheel. Excessive revolutions could cause the wheel to shatter.

6. Only use the _____________ of the wheel.

7. Feed the work into the wheel ____________________.

8. Shut off the machine immediately if the wheel begins to _________________ or vibrate.

9. Never use ________________ or hands to stop any grinder.



	WORD BANK: Stand, clearance, one-eighth, speed, face mask, condition, face, tools, gradually, chatter 




	HAND TOOLS SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Hand tools in _____________condition are responsible for many injuries.

2. After use, _____________and return the tool to its proper place.

3. All _____________should be removed before beginning work.

4. __________ are the cause of many accidents. Use only sharp tools that are in good condition.

5. Always push a chisel _______________from yourself.

6. Keep_____________ hands on the chisel, unless striking it with a mallet.

7. Use the_____________ tool for the job.

8. Always use a file with a _____________. Protect your hand from serious injury.

9. Wear ______________________________whenever you use striking tools.

10. Never __________________behind a person swinging a hammer.



	WORD BANK: 

eye protection, both, blunt cutting tools, handle, stand, poor, clean, away, jewellery, proper


	METAL LATHE SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always wear ________________________ when operating the engine lathe.

2. Remove _________________ and tie back ______________ hair.

3. Ensure you are familiar with _________________ and ___________________

of ON/OFF and Emergency Stop switches.

4. Make all adjustments/measurements to tools and workpieces with the ____________ off.

5. Remove the ___________________ before starting the lathe.

6. Check the tool bit and _______________ for security before operating the machine.

7. Make sure the _______________ is sharp and has proper clearance before beginning lathe work...

8. Never leave the lathe running _____________________.

9. Keep your ______________ away from chuck until it has come to a complete stop.

10. Remove metal chips and cuttings with a ___________________.



	WORD BANK:

Safety glasses, jewellery, long, brush, workpiece,  fingers, chuck key, power, location, operation, tool bit, unattended




	PLC SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Minimum PPE for working with PLC’s are ________________________ safety glasses.

2. Ensure _________________ power is completely turned off when installing or replacing wiring.

3. Use wiring _________________ or solid wires for wiring to the PLC. 

4. Make all _________________________________________ to the PLC and its associated components with the power off.

5. Tightening terminal screws to the specified ___________________ will prevent damage and possible failure of the PLC. 

6. Excessive _____________________________ of wires or cables where they attach to the PLC may lead to wire failure and become an electrical hazard.

7. To avoid electrical shock, make sure that ___________________________ are in place and securely attached.



	WORD BANK:

external, adjustments/measurements, CSA approved, terminals, bending, torque, terminal covers




	PLASMA ARC CUTTER SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. A minimum _____________________ #5 should be used when plasma arc cutting.

2. ___________________ clothing should not be worn when plasma arc cutting because they are highly flammable.

3. The __________________ plume when plasma arc cutting is large and needs to be well ventilated.

4. Avoid cutting with the plasma arc cutter in _________________________ locations. 

5. PPE including shaded lens, gloves, leather apron and _______________________

are recommended for plasma arc cutting operations. 

6. Avoid using the plasma arc cutter in areas where _____________________ gases are stored.

7. Attach the _____________________ securely to the work piece or the work table with good metal to metal contact.

8. Do not connect work cable to material that will __________________ when the cut is complete.

9. _________________  hot metal or allow it to cool before handling or letting it touch combustible materials



	WORD BANK:

Work cable, shaded lens, fume, synthetic, combustible, damp or wet, hearing protection , quench, fall away




	ROBOTICS SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always wear ________________________ when working with robotic equipment.

2. Ensure that adequate _________________________ such as safety barriers or light   curtains are installed around the robot work area.

3. Ensure you are familiar with _________________ and ___________________

    of ON/OFF and Emergency Stop switches.

4. Inspect the robot on a _____________________basis to ensure that all moving parts are in good working order.

5. Remove the ___________________ before making adjustments to the robot or equipment or materials within the robot work area.

6. The robot workcell must ___________________ with applicable local and national standards.

7. Make sure the ________________________ limits for the robot are not exceeded.

8. Never leave the robot running _______________________________.

9. Operate robot in a ____________________________ sequence when running new or modified programs for the first time.



	WORD BANK:

Safety glasses, safeguards,  single step, comply, power, location, operation, regular, unattended, payload




	SHEET METAL TOOL SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Ensure the machine is ____________________ attached to bench top or floor.

2. ___________________________ are minimum PPE for sheet metal work. 

3. Cutting sheet metal often creates sharp edges. Wear _________________ and use caution when handling sharp edges of sheet metal.

4. Work areas need to be clean and free of ______________________. 

5. Ensure that that the tool is rated to cut the _____________________ of metal you’re using.

6. Use the _____________________ to cut only sheet metal. Cutting other materials such as wire can damage the machine, making it unsafe for future use.

7. Avoid using excessive _________________ when operating any sheet metal tool.

8. Ensure all ______________________ are in place and properly functioning.

9. Keep _______________________________________ away from working areas.

10. One person only should __________________________ a machine at any time.



	WORD BANK:

hands and feet, gloves, operate,  force, guards, safety glasses, gauge, debris, shear, securely




	SURFACE GRINDER SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always wear ____________________ when operating a surface grinder.

2. Operate the surface grinder only after you have received ___________________ and permission from the instructor. 

3. Always _______________________ test the grinding wheel before mounting it to the machine.

4. When ______________ a grinder always stand to one side of the _______________ and make sure no one is in line with the grinding wheel in case of breakage.

5. Ensure that the grinding wheel clears the _____________ before starting the grinder.

6. Ensure the wheel ___________________ is in place and it covers _______________ the wheel.

7. Ensure the ______________________________is on and holding the work piece by trying to remove the work.

8. Never attempt to _______________ the magnetic chuck or mount or remove work until the _________________ has stopped completely.



	WORD BANK:

Wheel, guard, safety glasses, instruction, clean, ring, starting, work, half, magnetic chuck, wheel




	VERTICAL MILL SAFETY QUIZ



	Name:_______________________________

Date:________________



	1. Always wear ____________________ when operating a vertical milling machine.

2. Ensure _______________ and _______________ are mounted securely before starting motor and making a cut. 

3. Remove chips and cut off material with a _______________ when the machine comes to a complete ___________________.

4. Work areas need to be clean and free of ______________________. 

5. When setting up or removing a _____________________________ from its holder proper precaution should be taken to avoid getting injured.

6. Keep __________________________ away from revolving parts.

7. One person only should __________________________ a machine at any time.

8. Never___________ or_________________ work until cutter is completely stopped.



	WORD BANK:

 cutter, adjust, brush, milling cutter, hands, safety glasses, stop,  work piece,  operate, debris, measure




	SECTION 4: SAFETY PASSPORTS


This section contains Safety Passports, which provide a means to track individual student safety knowledge and skills. These Safety Passports insure that students have passed the required safety tests and understand the safety procedures and rules specific to the tools and equipment. It is recommended that all teachers keep records of signed passports at all times.

Safety Passports may be signed by teachers, parents and students before working on any workshop machine or tool. Signing signifies completion of safety training and testing.  There are three variations; teachers may select the most appropriate method to suit their needs. Ensure that the selected passports meet board and school policies.

Safety Record Card: for individual student, records their proficiency rating for each machine on one sheet.

Safety Passport: Form 1: single sheet for individual student and machine, has signature area and note area to be used in student notebook

Safety Passport Form 2: sheets for individual students listing machines, for teacher record book

Safety Passport Form 3: individual machine for each individual student, has line for parent signature to be used as a safety reinforcement or authorization, (see principal for permissions)

NOTE:

All materials within this document are to be considered as suggestions and recommendations only.  These are not legal documents and are not to be considered as legal requirements or as official policy. OCTE or the individual contributors makes no claim to the accuracy or the completeness of the enclosed documents and accepts no responsibility for any damages pertaining to their use. Users of this document should not assume all warnings and precautionary measures are contained herein, that additional information or measures are not required, or that local by-laws, regulations or Board policies are explicitly included.

Please see specific equipment manuals for further safety information, as well as local, Board and school policies and regulations.

Sample Student Safety Record Card

	Student Information
	Levels Chart

	Name:

Student #:

Grade:

Course/Section:
	Rating 1: May set-up equipment only, Instructor must do the work. 

Rating 2: Use only with an Instructor’s assistance.  

Rating 3: Full use with an Instructor standing by to supervise. 

Rating 4: Full use of machine with an Instructor’s permission.

(Note: Lower levels can be upgraded to higher levels with further instruction, practice and proof of competence. All students must have Instructor’s permission before using any equipment.)

	Manufacturing Equipment


	Manufacturing Equipment



	Equipment
	Rate
	Sign
	Date


	Equipment
	Rate
	Sign
	Date

	Arbor Press
	
	
	
	Radial Drill Press
	
	
	

	SMAW Welder
	
	
	
	Squaring Shear
	
	
	

	GMAW Welder
	
	
	
	Metal Roller
	
	
	

	GTAW Welder
	
	
	
	Sheet Metal Brake
	
	
	

	Oxy-Acetylene Welder
	
	
	
	Tube Bender
	
	
	

	Plasma Arc Cutter
	
	
	
	Angle Grinder
	
	
	

	Beverly Shear
	
	
	
	PLC
	
	
	

	Drill Press
	
	
	
	Belt/Disk Sander
	
	
	

	Pedestal Grinder
	
	
	
	
	
	
	

	Vertical Bandsaw
	
	
	
	
	
	
	

	Horizontal Bandsaw
	
	
	
	
	
	
	

	Abrasive Cut Off Saw
	
	
	
	
	
	
	

	Engine Lathe
	
	
	
	
	
	
	

	Vertical Milling Machine
	
	
	
	
	
	
	

	Horizontal Milling Machine
	
	
	
	
	
	
	

	Surface Grinder
	
	
	
	
	
	
	

	CNC Milling Machine
	
	
	
	
	
	
	

	CNC Plasma Arc Cutter
	
	
	
	
	
	
	

	Robotics
	
	
	
	
	
	
	


Technology Lab Safety Passport

The purpose of the safety passport is to ensure that students are fully aware of all safety features on each piece of equipment in the technical facility prior to using them independently.  

The general process is as follows:

1.
Teacher Demonstration: When the teacher introduces a new piece of equipment, the student records the date of the safety demonstration on their safety passport. This is to be initialed by the teacher (see sample below). The teacher demonstrates techniques for the safe operation and procedures, as well as use of personal protective equipment (e.g. eye protection, secure loose hair, remove jewelry, protective clothing, etc.). Students prepare notes in their notebooks. This safety note is carefully recorded in each student’s notebook along with the signed passport. The teacher also carefully notes attendance for that day in their daybook if any students are absent for the safety lesson; makeup opportunities must be provided.


2. Test: Each student should complete a written (or oral) test on the safe operation or procedure, outlining all safety features that must be observed. The individual tests are designed to complement any general facility safety rules. Upon satisfactory completion of the test the student dates the “tested” column and teacher initials this as complete. IMPORTANT NOTE: A copy of the test should be kept by the teacher.
3. Student Demonstration: Students must demonstrate to the teacher that they have a thorough knowledge of the safety rules for the equipment and are able to demonstrate their competency on the equipment. Once the teacher has observed the required safe setup and operation of the equipment by a student the teacher signs off that portion of their passport. 

4. Once the student has completed #1, 2 and 3, the teacher signs the final column of student’s safety passport indicating they have permission to use that equipment or perform the procedures. Students must be able to provide the teacher with their signed passport for that equipment each time they wish to use that equipment.  

Note: Three forms are provided, Form 1 can be used as a student notebook form for each machine; Form 2 can be used for signing several machines per student. With the 2nd form, students keep safety notes on separate paper. The third form requires one sheet per tool per student, and may be used in the student notebook or kept on file by the teacher (or both).

Safety Passport Form 1

Student Name:
__________________ Course/class:  

	Equipment:
_____________________________



	Attended Teacher Safety Instruction and Demonstration

(notes recorded)
	Passed Written or Oral Testing
	Demonstrated Safe Setup and Operation of Equipment to Teacher
	Granted Permission to use Equipment by Teacher

	Date of

Lesson
	Teacher Initial
	Date 

Tested
	Teacher Initial
	Date of Demo.
	Teacher Initial
	Date
	Teacher

Initial

	
	
	
	
	
	
	
	


NOTES:

Safety Passport Form 2

Student Name:
______________________ Course/Class: 


	Equipment/Procedure: _____________________________


	Attended Teacher Safety Instruction and Demonstration

(Notes recorded)
	Passed Written or Oral Testing
	Demonstrated Safe Set-up and Operation to Teacher


	Granted Permission by Teacher

	Date of

Lesson
	Teacher Initial
	Date 

Tested
	Teacher Initial
	Date of Demo.
	Teacher Initial
	Date
	Teacher

Initial

	
	
	
	
	
	
	
	


	Equipment/Procedure: _____________________________


	Attended Teacher Safety Instruction and Demonstration

(Notes recorded)
	Passed Written or Oral Testing
	Demonstrated Safe Set-up and Operation to Teacher


	Granted Permission by Teacher

	Date of

Lesson
	Teacher Initial
	Date 

Tested
	Teacher Initial
	Date of Demo.
	Teacher Initial
	Date
	Teacher

Initial

	
	
	
	
	
	
	
	


	Equipment/Procedure: _____________________________


	Attended Teacher Safety Instruction and Demonstration

(Notes recorded)
	Passed Written or Oral Testing
	Demonstrated Safe Set-up and Operation to Teacher


	Granted Permission by Teacher

	Date of

Lesson
	Teacher Initial
	Date 

Tested
	Teacher Initial
	Date of Demo.
	Teacher Initial
	Date
	Teacher

Initial

	
	
	
	
	
	
	
	


Safety Passport Form 3
Equipment/Procedure Passport

	[EQUIPMENT/PROCEDURE]

	General Conditions



	Personal Protective Equipment 



	Possible Risk Factor


	· The student has been trained on this equipment and procedure.

· The student understands the required personal protective equipment to operate this               equipment and perform this procedure.

· The student is aware of the possible risk factors


Student signature

________________________________


Teachers signature
________________________________


Date of training

________________________________



	


	ARBOR PRESS PASSPORT

	General Conditions

Students must be trained on the safe use of an Arbor Press before they may begin using it. The student must demonstrate to the teacher proficiency and the safe work procedures that must be followed before usage.



	Personal Protective Equipment 

· Full face shield

· Safety glasses

· Coveralls or apron

· Safety footwear



	Possible Risk Factor
· Small projectiles 

· Slips and falls 

· Strain injuries

· Impact injuries [parts breakage]



	· The student has been trained on this equipment.

· The student understands the required personal protective equipment to operate this equipment.

· The student is aware of the possible risk factors


Student signature

________________________________


Teachers signature
________________________________


Date of training

________________________________




	ERGONOMICS PASSPORT

	General Conditions

Improper posture, equipment placement, and repetitive use of equipment may cause injuries and pain. Students must be trained on the safe and proper use of equipment before they may begin using them. The student must demonstrate the ability to use the equipment safely.



	Personal Protection
· Proper posture

· Proper equipment placement

· Change in sitting arrangements, etc. to avoid repetitive stress injuries



	Possible Risk Factor
· Spine and back injuries

· Hand Injuries

· Eye strain


	· The student has been trained on this equipment.

· The student understands the required personal protective equipment to operate this equipment.

· The student is aware of the possible risk factors


Student signature

________________________________


Teachers signature
________________________________


Date of training

________________________________




	INTERNET USE PASSPORT

******TO BE USED AS AN EXAMPLE ONLY – PLEASE SEE BOARD/SCHOOL POLICY*****

	General Conditions

Students must be trained on the safe and proper use of the Internet before they may begin using it.  The student must demonstrate to the teacher, knowledge of safe and secure procedures as outlined in the Internet Use Policy Document.  

	Personal Protection
· Knowledge of school and school board Internet Use Policy 

· Never releasing personal information

· Avoidance of insecure and questionable sites

· Respect for self and others

· Awareness of security issues in communications technology



	Possible Risk Factor
· Threats to personal safety and/or security

· Loss of privacy


· Threats to emotional security

· Spread of damaging computer viruses

· Damage to computer operating and networking systems



	· The student has been trained on this equipment.

· The student understands the required personal protective equipment to operate this equipment.

· The student is aware of the possible risk factors


Student signature

________________________________


Teachers signature
________________________________


Date of training

________________________________




 Sample: Record of Safety Training 

Student:


Class:

Over the course of the semester or term(s) you will receive direct instruction in the safe and appropriate use of the all the equipment, tools, materials, and facilities required to complete your classroom activities. Instruction consists of a combination of demonstration and written and verbal instruction. A satisfactory mark on a safety quiz following the instruction demonstrates the acquisition of sufficient knowledge to use and access the relevant equipment and materials. Your ongoing demonstration of safe practice is assessed in the project marking. Your teacher will put the date and sign-off beside each topic in acknowledgement of your attendance at the discussion or demonstration.

STUDENTS MAY NOT USE ANY EQUIPMENT, TOOL, OR FACILITY UNTIL:

· his or her training has been signed off by the teacher

· he or she has received a satisfactory mark on the related safety quiz.

	Topic
	Date
	Teacher’s Signature

	Computer Resources and the Internet

	· Acceptable Use Policy

· Safety on the Internet

· Computer Ergonomics
	
	

	Patient or Client Care

	· Safe use of chemical treatments

· Use of personal protective equipment (PPE) for patient/client 

· Safe and proper handling of patient or client

· 
	
	

	Facility Care

	· Proper cleaning and setup procedures

· Maintaining safe working environment

· Use of personal protective equipment (PPE) for self 

· Proper sanitation and sterilization procedures

· Safe and proper disposal of consumables and hazardous materials


	
	


	WHMIS Regulations



	· The acronym WHMIS stands for Workplace Hazardous Materials Information System
· This regulation was introduced into the Occupational Health and Safety Act of Ontario in 1987

· WHMIS gives a worker the right to know what they are handling

· Federal WHMIS applies to importers, manufactures, and suppliers of hazardous materials

· The Hazardous Products Act for controlled products states that national suppliers are to provide labels and Material Safety Data Sheets to all buyers

· MSDS stands for Material Safety Data Sheets
· MSDS is a print out on paper that identifies how to handle, store, use, health effects if exposed, emergency procedures, and protective measures

· MSDS must be updated by the supplier every 3 years
· Under the Occupational Health and Safety Act in Ontario, employers are to make sure that all hazardous materials are labeled appropriately and that a MSD sheet accompanies the product

· On any controlled hazardous product two labels must be properly labeled with a supplier and a workplace sticker

AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR

                  Certification Requirements



	     Welder certification is based on specially designed tests to determine a welder's skill       and ability to deposit sound weld metal. The welder's tests consist of many variables, including the specific welding process, type of metal, thickness, joint design, position, and others. Welder qualification is carried out according to CSA Standards and ASME.[1] The ASME code is typically used for pressure vessel and pressure piping applications, and CSA Standards are used for structural, general manufacturing and non-pressure applications. There are 3 major CSA Standards to which welders may be qualified: CSA W47.1[3] for steels (including stainless steels), CSA W47.2[4] for aluminum, and CSA W186[5] for reinforcing bars. Under these CSA standards, welder qualification testing is carried out every 2 years by the Canadian Welding Bureau to ensure ongoing competence.

CWB approved companies or private individuals, with proof of one year's work experience in welding, can arrange for shielded metal arc weld testing in accordance with CSA W47.1/W59 standards. Welds are witnessed and X-ray tested by CWB. Positions include Flat, Horizontal, Vertical and Overhead.

Individuals who are not employed by a company certified to CSA W47.1 and are not a member of a union providing testing facilities, can arrange to be tested at an authorized Canadian Welding Bureau Test Centre.

Welding procedures and practical testing through CWB links on youtube: www.youtube.com/watch?v=yukp9290KbM
AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR


	SECTION 5: EMPHASIS COURSE RESOURCES


TMM3M/TMM4M – Mechanical Engineering

TMR3M/TMR4M – Robotics and Control Systems

https://www.osha.gov/dts/osta/otm/otm_iv/otm_iv_4.html
TMC3C/TMC4C – Computer Aided Manufacturing

http://tdserver1.fnal.gov/tdweb/ms/Policies/CNC_guidelines.pdf
http://server2.smithy.com/media/pdf/CNC%20Machine%20Safety.pdf
TMI3C/TMI4C – Industrial Maintenance

http://industrialplantsafety.com/
http://www.idc-online.com/technical_references/pdfs/instrumentation/PLC_Startup_main.pdf
TMP3C/TMP4C – Precision Machining

http://cte.unt.edu/content/files/_MFG/MFG_Curriculum/Flexible_Mfg/03GeneralSafety/03.03PrecisionMachiningSafety.pdf
TMT3C/TMT4C – Robotics and Control Technician

https://www.osha.gov/dts/osta/otm/otm_iv/otm_iv_4.html
TMY3C/TMY4C – Welding Technician

http://www.aws.org/technical/facts/
http://www.lincolnelectric.com/en-ca/education-center/welding-safety/pages/welding-safety.aspx
http://www.millerwelds.com/resources/safetyresources.html
www.cwa-acs.org- Canadian Welding Association
  
TMO3E/TMO4E – Machine Operator

http://server2.smithy.com/media/pdf/CNC%20Machine%20Safety.pdf
TMS3E/TMS4E – Sheet Metal

http://www.ihsa.ca/resources/health_safety_manual/pdfs/trades/Sheet_Metal.pdf
TMW3E/TMW4E – Welding

http://www.aws.org/technical/facts/
http://www.lincolnelectric.com/en-ca/education-center/welding-safety/pages/welding-safety.aspx
http://www.millerwelds.com/resources/safetyresources.html
www.cwa-acs.org- Canadian Welding Association
	APPENDIX A: HEALTH AND SAFETY RESOURCES


Work Smart Ontario! 

http://www.worksmartontario.gov.on.ca
Is the official website of the Ontario Ministry of Labour for young workers and new workers.  Utilize this website to find out how to be safe at work.  Find out how to be treated fairly!  Includes key information on: My Health and Safety at Work, My Employment Standards and I’ve Got a Problem – What Do I Do Now?

Workplace Safety and Insurance Board

http://www.wsib.on.ca
Legislated by the Ontario government and responsible for administering the Workplace Safety and Insurance Act (WSIA). Governed by a Board of Directors made up of representatives of workers, employers and others.

Under the Resources tab, this website provides information on how WSIB make decisions, by reviewing the Operational policy manual, Employer Classification Manual, and Adjudication support documents. You’ll also find useful forms and fact sheets on a variety of topics, including benefit payments, and rights and responsibilities.  

· Fact Sheets are also available:

· Fact Sheets for Workers

· Fact Sheets for Prevention

· WSIB Fact Sheets


Workplace Safety Resources Inc. 

http://workplacesafetyresources.ca
This site provides a personalized approach to planning for safety.  Workplace Safety Resources Inc.’s mission is to create healthy, safe, secure and environmentally responsible workplace, to work with industry to better protect all employees, to improve the quality of life in workplaces and communities and become a recognized leader in providing effective safety programs, products and services for the prevention of injury and illness.

Take Our Kids to Work – Teacher’s Guide; Workplace Guide

The Learning Partnership 
http://www.tlp.on.ca
These resources have been custom designed to help teachers and workplaces prepare for Take Your Kid to Work day.  The new booklets have an excellent section on activities to help prepare the students for a safe learning day.

Canadian Centre for Occupational Health and Safety

http://www.ccohs.ca/resources
The Free Resources section is a collection of websites, databases, and other online resources suggested and reviewed by CCOHS. Many of the websites are designed and maintained by CCOHS, while some of the resources are provided by external, third-party providers.

Purpose

· Promote the importance of workplace health and safety in Canada

· Identify current and reliable health and safety information

· Create and maintain an accessible, convenient, and easy-to-use resource to anyone who needs it

· Provide access to information from a variety of sources including federal, provincial, and territorial governments, agencies, and non-profit organizations

Target Audience

The Free Resources are useful to workers, employers, managers and supervisors, joint health and safety committees, workplace health and safety professionals, and students.

HEALTH CANADA

http://www.hc-sc.gc.ca   

Health Canada is the Federal department responsible for helping Canadians maintain and improve their health, while respecting individual choices and circumstances. 

Health Canada administers many pieces of legislation and develops and enforces regulations under this legislation that have a direct impact on the health and safety of Canadians. The Department consults with the Canadian public, industry, non-governmental organizations (NGOs) and other interested parties in the development of these laws. Health Canada also prepares guidelines in order to help interpret and clarify legislation and regulations.

Of particular interest would be regulations such as the Hazardous Product Act, Controlled Products Regulations, Environmental and Workplace Health.


HEALTH & SAFETY ONTARIO (HSO)
http://www.healthandsafetyontario.ca/HSO/Home.aspx 

Ontario is already a great place to do business, live and work. Making our province, and indeed our country, the healthiest and safest place to work in the world is a prize worth winning. 

Ontario’s Prevention System is made up of the Ministry of Labour (MOL), Workplace Safety and Insurance Board (WSIB), Workers Health & Safety Centre, Occupational Health Clinics for Ontario Workers Inc. and 12 Health and Safety Associations (HSAs).

Health & Safety Ontario (HSO) is the result of a bold move to reorganize the independent efforts of 12 health and safety associations into four streamlined organizations to better serve more than 236,000 Ontario businesses. 

HSO is comprised of:
· Workplace Safety & Prevention Services
· Public Services Health & Safety Association 
· Workplace Safety North 
· Infrastructure Health & Safety Association.
ONTARIO BUILDING CODE

http://www.mah.gov.on.ca/Page7393.aspx 

The Ontario Building Code’s website has information on qualification and registration, available training, dispute resolution, news on recent code developments and more. The Ontario Building Code is administered by the Building and Development Branch of the Ministry of Municipal Affairs and Housing. 

CANADIAN STANDARDS ASSOCIATION (CSA)

http://www.csagroup.org 

Standards contribute to safer homes, workplaces and public spaces. They address issues related to sustainability and the environment. And they encourage the adoption of new technologies and best practices that enhance trade and help make industry more competitive in the global marketplace. Standards help advance today, while anticipating tomorrow.

CANADIAN SOCIETY OF SAFETY ENGINEERING (CSSSE)

http://www.csse.org/ 

The Canadian Society of Safety Engineering (CSSE) is the leading health, safety and environmental organization for professionals in Canada. They work with industry, governmental agencies, and other safety organizations to promote a greater awareness of health, safety, and environmental issues in workplaces and communities across the nation and around the world. Our vision is "An Advocate for Safety in Every Workplace".

CSSE’s mission is to be the resource for professional development, knowledge and information exchange to our members, and the Canadian public.
CANADIAN WELDING ASSOCIATION- www.cwa-acs.org
Provides a Canadian forum in which welding professionals, their employers, fabricators and welding-products suppliers are all able to exchange information, share knowledge resources and work with government and regulatory agencies and each other to advance the welding profession, its products and its practices.
PROFESSIONAL ASSOCIATIONS
Professional Associations can be a great health and safety resource relating to discipline specific occupational health and safety. The following Tech Design related associations provide resources on professional practice relating to health and safety.
Professional Engineers of Ontario (PEO)…… www.peo.on.ca/
Architectural Association of Ontario (OAA)….. www.oaa.on.ca/ 
Ontario Certified Engineering Technicians and Technologists (OACETT)….. www.oacett.org/ 
Association of Registered Interior Designers of Ontario (ARIDO) ….. www.arido.ca/ 

The Canadian Welding Association…. www.cwa-acs.org
Live Safe! Work Smart! Grade 9/10 and Grade 11/12 

http://www.livesafeworksmart.net 

Written by health and safety professionals, produced by the Ministry of Labour in partnership with the Ministries of Education and Training, Colleges and Universities, Live Safe! Work Smart! provides the only comprehensive resource for Ontario teachers developed to match health and safety curriculum expectations from Grades 9 – 12.  It has received top marks from the Ontario Curriculum Center.  Reviews can be viewed on the OCC website: (www.curriculum/occ/resources.org).  Within the two-volume set are lessons, overheads, handouts and exercises well suited to cooperative education and apprenticeship.  

Binders and CDs have been distributed to all secondary schools in Ontario.  If you can’t locate either, check the website at www.livesafeworksmart.net for who to contact in your Board to get more information on your local resources, or to order a CD of your own.  If you don’t have access to the web, you can place an order by calling 1-800-268-8013.

http://www.edu.gov.on.ca/eng/studentsuccess/pathways/files/septNews.pdf
http://www.livesafeworksmart.net/english/grade%209-12/course_streams.htm
http://www.livesafeworksmart.net/english/special_needs/index.htm
Ontario Ministry of Labour

Web address: http://www.labour.gov.on.ca/english/
For news and information about Ontario’s health and safety and employment legislation, the Ministry of Labour’s website is an excellent place to visit.  It provides current information on both employment standards and health and safety legislation, recent fines, alerts, etc. and allows you to ask a question that will be answered by Ministry staff.  To directly access information for students, use the web address: http://www.worksmartontario.gov.on.ca/scripts/default.asp  or http://www.labour.gov.on.ca/english/es/pubs/factsheets/fs_young.php
This section of the Ministry of Labour website ensures that students are aware of their rights and obligations and their employer’s rights and obligations under the Occupational Health and Safety Act and the Employment Standards Act.  It includes: young worker safety education information; information for working students – know your rights and obligations; information for new workers and students working in Ontario; fact sheets for employees; your guide to the Employment Standards Act; and links to related websites.


Canadian Welding Bureau

Canada, like many countries around the world, has specific rules and regulations governing the construction of buildings and their related structural components. In Canada, companies doing such work are governed by the National or Provincial Building Code(s) and CSA Standard S16 Design of Steel Structures. These codes and standards are mandatory and apply to all Canadian welding, fabricating or erection companies, as well as US or International companies, working with or providing  applicable structural welded products for use within Canada. In addition to the above, there are also several additional product and safety codes that include requirements for CWB certification.
Workplace Safety and Insurance Board

http://www.wsib.on.ca 
Contains information for both employers and employees about workplace safety. Includes advice on prevention, important news releases, policies and other work-related information. 

http://www.wsib.on.ca/en/?in_tx_query=students&in_hi_space=SearchResult&in_hi_control=bannerstart&cached=false&in_hi_req_apps=1&in_hi_req_objtype=18&in_hi_spell=1&in_hi_req_ddfolder=595&in_hi_req_subfolders=1&num=25&search.x=57&search.y=15
http://www.wsib.on.ca/en/community/WSIB/230/ArticleDetail/24338?vgnextoid=52bfe1880bc5c210VgnVCM100000469c710aRCRD&vgnextchannel=a7620b368d5dd110VgnVCM1000000e18120aRCRD 

http://www.wsib.on.ca/en/community/WSIB/230/ArticleDetail/24338?vgnextoid=8f7d3260ffc88210VgnVCM100000469c710aRCRD
Ontario School Boards Insurance Exchange

http://www.osbie.on.ca 
The primary goals of the Exchange are to insure member school boards against losses, and to promote safe school practices. The Ontario school “Risk Management at a Glance” material is intended to provide guidance and direction in the major risk management areas facing school administrators, principals, vice-principals, teachers and all other school staff on a daily basis.

http://www.osbie.on.ca/risk-management/
http://www.osbie.on.ca/risk-management/#resources
http://www.osbie.on.ca/risk-management/presentations/presentation-form.aspx
Although this reference material is not intended to replace school board policies and procedures, it is intended to supplement the risk management considerations, which should go into making the decisions on the most common day-to-day school activities. The design of this publication is to promote the display of this document in a calendar-like format in every classroom to facilitate ready “Risk Management at a Glance”. Every employee who may be called upon to make a decision about the permitting of or the organizing of any activity listed can use this.

For any activities not listed in this material, it is recommended that you contact your board office, or refer to the policies and procedures as stated by your school board.
	APPENDIX B: OCTE SAFETYNET BLANK TEMPLATE


Overview

A sample of a blank SafetyNET template provided by the Ontario Council for Technology Education as well as their Materials and Resources sheet has been provided here as an additional resource for computer technology teachers. 

Completing it once for a risky project can take teachers through a pre-project planning process, a review of the materials in their shops, the suppliers and processes they use, and encourage documentation of their safety training for themselves, their students, and classrooms. It collects safety information in one place for their own use, and respects their experience, pedagogy, and professionalism. It’s a crucial step in standardizing safety training in your technology program at your school, and can assist in collegial communication in your department.

Please note that the online updated version is available at www.octelab.com, however any teacher that considers and documents their answers to the questions will have created an important document for their personal professional practice. It’s also available in fillable .pdf format, and is also available in French from OCTE

Establishing A Safety Binder

The goal is a safety binder that teachers keep in their rooms as evidence of due diligence taken towards safety in the classroom. 

Assembled safety binders often include teacher/room/board specific:

· •SafetyNET Template

· •Project Specific Safety Resources

· •MSDS Sheets

· •Student Safety Training Tracking Sheets

· •Permission Forms Copies

· •Class Lists

· •Equipment Maintenance/Manuals

· •Training Quiz Samples

· •Teacher Training Documentation Copies

· •Emergency Procedures Docs

· •Board Repair Contacts
·  Room Safety / PPE Location Map

Starting Your SafetyNET 

TMJ Subject Area:  Tech department heads can provide leadership asking teachers to consider the following questions to choose a focus for completing their own SafetyNET. 

· What are the most risky projects I do in my classroom? (List them here.) 

· What ones of these use the riskiest materials?  

· Which ones of these use the highest risk-associated equipment?  

· Which ones of these include recycled, found, repurposed, or donated materials? 

· Which one of these is the hardest to train and track the kids for safety on?  

· Reflecting on this listing, which project do you think you may want to do a SafetyNET on?  

· What resources of mine would make it easier - instructive for another teacher to try this project?  

· What would be the best “safety lens” advice I could give for another teacher from my experience?

Then try it out!
	SafetyNET Lesson Plan 


SafetyNET STEP 1: Tell Us About You

First Name: _________________________________

Last Name: _________________________________

E-mail Address: ______________________________

Ontario School Board: _________________________

School: _____________________________________

Community 

[image: image10.wmf]Urban

[image: image11.wmf]Suburban

[image: image12.wmf]Rural

Number of Students:

Student Work is Completed (individually, pairs, groups, mixed methods)

[image: image13.wmf]

Mixed

 

Methods


[image: image14.wmf]
I agree to the Terms and Conditions and have read the Teacher Guidelines.

SafetyNET STEP 2: Describe Your Lesson
Classroom Management Pre-Planning
1. Provide a descriptive title for your learning activity. 

[image: image15.wmf]


2. Choose the length that best describes your lesson.

[image: image16.wmf]Full semester

[image: image17.wmf]Multiple weeks

[image: image18.wmf]One week

[image: image19.wmf]One period

3. Choose the Ontario course code (e.g.). 

[image: image20.wmf]

TEJ

 

-

 

Computer

 

Engineering


4. Provide learning goals of the activity. 

Names of Resource Files Included: (Please format as .pdf where possible.)

5. Generally describe your classroom lab setup with main equipment and areas.

6. There is a link here to your subject area's full Overall and Specific required Ministry Expectations. Click here for safety expectations summarized for each tech course code. These will create a pop-up window for copying and pasting into the field below. Copy and paste some safety expectations your lesson will cover.

7. There may also be local by-laws or staff guidelines applicable to your school community in general that affect how you teach your subject area for health and safety. Being in an urban or rural environment can offer unique challenges to a technological education program. Your department or school may also have a health and safety manual you can attach as a file later. Include any details or best practices here on what you refer to.

8. Coming from industry and experience as a technological educator, there is prior teacher knowledge that you would recommend for your classroom, focused on health and safety. Include information on recommended certifications for your subject area.

9. Many teachers use these as a basis of training for prior student knowledge. Check off which ones you use currently. A pop-up window is available through these links.

[image: image21.wmf]Passport to Safety

[image: image22.wmf]Introduction to WHMIS

10. Prior to specific project work, describe your general introductory unit on health and safety in your classroom.

11. Check off what Personal Protective Equipment may be applicable in your classroom in general for health and safety. 

[image: image23.wmf]safety glasses (shatterproof - may need side guards)

[image: image24.wmf]coveralls / lab coat / apron (protective clothing)

[image: image25.wmf]gloves (latex and standard)

[image: image26.wmf]gloves (chemical resistant)

[image: image27.wmf]welding gloves and face shield

[image: image28.wmf]dust mask (breathing protection)

[image: image29.wmf]respirator (breathing protection)

[image: image30.wmf]appropriate footwear (may imply steel-toed work boots or closed toe and heel shoes)

[image: image31.wmf]hair net

[image: image32.wmf]hair tied back

[image: image33.wmf]hearing protection - ear plugs

[image: image34.wmf]removing jewelry and fashion accessories

[image: image35.wmf]hard hat

[image: image36.wmf]safety harness

[image: image37.wmf]reflective vest

[image: image38.wmf]no electronic devices

12. Describe your student safety training assessment strategies. Click here for a pop-up to review the Growing Success document that defines assessment for learning and as learning. 

13. Some technological classroom areas are more complex and need layout planning, maintenance, and special resources available, especially when sharing rooms. Detail general housekeeping, organization standards and student clean-up procedures from your experience. 

14. Detail safe storage facilities in your classroom for course specific materials.

15. Explain any special learning considerations and best practices for your classroom focused on safety. Are there left-handed students in your class? You may naturally include accommodations and modifications. Showcase special approaches or methods you use for exceptional students, multiple-intelligences, differentiated instruction, ESL, gifted, or physically-challenged students.

16. Include information on your safety procedures for disposal of waste materials. This could include food scraps, hairstyling chemical, dust collection, combustible wipes, or waste oil.

17. Company's coming! Educational Assistants, volunteers, student teachers, and classroom guests with administrators are in your classroom. Provide your experience on elements of safety training that need to be communicated to these participants for your subject area such as wearing safety glasses, maintaining distance from machines, or how to communicate an emergency or issue to the teacher. 

18. Emergency procedures to pre-plan in general for your technological education classroom depends on your subject area. There may be steps for students, steps for administration, for assisting teachers, or directions for emergency assistance arriving at school. Detail how you cover these in your classroom. Include fire exits, extinguishers, first aid station, eye wash station, and electrical shut-off switches (panic buttons). Possibly detail AED location (if available) and first aid trained staff member locations for your records. 

19. Does your Board have a technological project approval process?

[image: image39.wmf]Yes

[image: image40.wmf]No

[image: image41.wmf]Unknown
20. Select (all that apply) that complete equipment inspections in your board.

[image: image42.wmf]Teacher

[image: image43.wmf]Department Head

[image: image44.wmf]Board Instructional / Subject Area Leader

[image: image45.wmf]Board Facilities Teams

[image: image46.wmf]Independent Contractors

[image: image47.wmf]Ministry of Labour

21. Select Federal and Provincial Safety Legislation and Policies, Government Departments, and Associations which may be applicable to your subject area. Click on any of them to open up a pop-up window to reference their website. Consider adding any resources you find to your lesson. 

[image: image48.wmf]Health Canada

[image: image49.wmf]Ministry of Labour

[image: image50.wmf]Ontario Workplace Safety and Insurance Act

[image: image51.wmf]Food Safety and Quality Act

[image: image52.wmf]Ontario Health Protection and Promotion Act

[image: image53.wmf]Ontario Highway Traffic Act

[image: image54.wmf]Ontario Fire Code

[image: image55.wmf]Ontario Building Code

[image: image56.wmf]Workplace Hazardous Materials Information System (WHMIS)

[image: image57.wmf]Workplace Safety and Insurance Board (WSIB)

[image: image58.wmf]Occupational Health and Safety Act (OSHA)

[image: image59.wmf]Apprenticeship and Certification Act (ACA)

[image: image60.wmf]Canadian Standards Association (CSA)

[image: image61.wmf]Canadian Society of Safety Engineering (CSSE)

[image: image62.wmf]Ontario Service Safety Alliance (Hospitality and Tourism) (OSSA)

[image: image63.wmf]Canadian Centre for Occupational Health and Safety (CCOSH)

[image: image64.wmf]Construction Health and Safety Association of Ontario (CSAO)

[image: image65.wmf]Ontario School Boards Insurance Exchange (OSBIE)

[image: image66.wmf]Industrial Accident Prevention Association (IAPA)

[image: image67.wmf]Transportation Health and Safety Association of Ontario (THSAO)

[image: image68.wmf]Health Care Health & Safety Association of Ontario (HCHSA)

That's the end of general classroom management info. You can copy and paste the content from this section to any project you submit to the SafetyNET. 

That's So Cool! When Do We Start? 
22. Check off planning tasks you complete for this lesson. 

[image: image69.wmf]examine materials list (new, used, recycled materials)

[image: image70.wmf]review tool use plan (power and hand tools)

[image: image71.wmf]consider special preparation of recycled materials for this project.

[image: image72.wmf]review hazardous materials use - WHMIS, MSDS (attach files later)

[image: image73.wmf]safety check on specific equipment

[image: image74.wmf]review chemical and fire safety procedures

[image: image75.wmf]prepare tools

[image: image76.wmf]count or measure materials, evaluate efficiencies

[image: image77.wmf]check 'past due' dates on supplies

[image: image78.wmf]check student-accessible material supply areas are safe

[image: image79.wmf]re-do a safety demonstration

[image: image80.wmf]confirm all students completed training diagnostic assessment

[image: image81.wmf]confirm web resources and handouts are current

[image: image82.wmf]reconsider assessment and evaluation strategies

[image: image83.wmf]plan direct supervision time for difficult or high-risk production steps

[image: image84.wmf]plan direct supervision for flammable / toxic / corrosive materials handling

[image: image85.wmf]plan safe storage of in-progress student projects

[image: image86.wmf]plan cut off times for lab cleanup to begin

[image: image87.wmf]plan waste disposal, recycling

[image: image88.wmf]plan debrief on safety risk experiences with students

[image: image89.wmf]detail notes for teacher sharing classroom/lab

23. The Ontario Skills Passport identifies working safely as a work habit. Check off the skills relevant for this lesson here. For more information click here to go to their website.

[image: image90.wmf]Work Habits: Working Safely

[image: image91.wmf]Work Habits: Teamwork

[image: image92.wmf]Work Habits: Reliability

[image: image93.wmf]Work Habits: Organization

[image: image94.wmf]Work Habits: Working Independently

[image: image95.wmf]Work Habits: Initiative

[image: image96.wmf]Work Habits: Self-advocacy

[image: image97.wmf]Work Habits: Customer Service

[image: image98.wmf]Work Habits: Entrepreneurship

[image: image99.wmf]Essential Skills: Reading Text

[image: image100.wmf]Essential Skills: Writing

[image: image101.wmf]Essential Skills: Document Use

[image: image102.wmf]Essential Skills: Computer Use

[image: image103.wmf]Essential Skills: Oral Communication

[image: image104.wmf]Numeracy: Money Math

[image: image105.wmf]Numeracy: Scheduling or Budgeting and Accounting

[image: image106.wmf]Numeracy: Measurement and Calculation

[image: image107.wmf]Numeracy: Data Analysis

[image: image108.wmf]Numeracy: Numerical Estimation

[image: image109.wmf]Thinking Skills: Job Task Planning and Organizing

[image: image110.wmf]Thinking Skills: Decision Making

[image: image111.wmf]Thinking Skills: Problem Solving

[image: image112.wmf]Thinking Skills: Finding Information

24. The Ontario Skills Passport (OSP) National Occupation Classification (NOC) code is a number that Human Resources and Skills Development Canada (HRSDC) has assigned to a particular occupation. To make safety training more relevant to students, check here and copy an example of career choice that has to be aware of the same classroom safety requirements.

25. Detail instructional strategies and assessment strategies for focusing on safety during this learning activity. Consider any IEP considerations applicable in your classroom.

26. Define the materials and equipment used for this learning activity. You can use the blank form that's provided here and save it to make it your own. The layout helps you collect details showing the materials and equipment. It also provides space for equipment maintenance schedules, disposal of waste materials, training tracking, shielding or guarding details. 

27. Include any best practices or tips, tricks, and advice in your experience of completing this learning activity. Focus your answer on how you document safety training, and share information about your shop with other tech teachers. That’s an OCTElab SafetyNET!

28. Provide a short description of your project that can go with a reference image for the database. (Max 256 characters.) 

SafetyNET STEP 3: Add Files and Videos
Please attach a project image for us to display with your short description in the database. Please upload any supporting documents including safety components, lesson materials, assessment tools, digital resources, images, or videos.  To bring your lesson to life, include online videos URL link files on the lesson plan page. Add as many as you like. 

Do you have a safety features map of your classroom you can share? Attach it here! 

Find the Material Safety Data Sheet (MSDS) for any of your materials clicking and searching here. Save it and add it to your digital resources to attach with your lesson.

The Ontario Ministry of Labour makes available a resource named Live Safe! Work Smart! It has a wide range of general safety and subject-specific resources available for use in the classroom and beyond. Click here to open a pop-up and copy and paste links that are your favorites here or download a resource you can use with this lesson and attach it later. You can also add any other URL links here that you think enhance this safety learning activity. 

SafetyNET STEP 4: Tag Your Lesson
Add your own descriptive tag(s) to help users search for content like yours. Print your lesson to document your SafetyNET for your classroom. Submit your SafetyNET lesson.  Plan to update lesson content or add digital resources later with your user login. Think about adding another lesson!  Remember, most of your general classroom info is already in.  You can 'Save As' and 'Modify' to submit a new lesson with new resources!

OCTElab SafetyNET – Materials, Physical Resources Planning Sheet

Teachers can copy and add rows to this blank form to address specific project needs and include it in their safety binder.

PROJECT / LEARNING ACTIVITY TITLE:
 
COURSE CODE AND TITLE: 



VERSION PREPARED DATE: 



SUBMITTED BY: 




CONTACT:  




MATERIALS LIST 

	MATERIAL
	QUANTITY
	DESCRIPTION
	SOURCE
	WHMIS
MSDS ATTACHED
	SAFE STORAGE 
	WASTE DISPOSAL

	
	
	
	[     ] new, purchased

[     ] new, donated from community, industry

[     ] recycled from inside school

[     ] recycled from outside school

PREPARATION REQUIRED FOR USE:

DETAILS:


	[    ] Y

[    ] N


	
	


PHYSICAL RESOURCES USED 

	EQUIPMENT, TOOL, MACHINE
	SUBJECT – SPECIFIC NEEDS
	INSPECTED FOR SAFETY FEATURES
	STUDENT TRAINING PLAN IDENTIFIED
	MAINTENANCE
SCHEDULE

	NOTE: TEACHER EXPERIENCE AND SAFETY PROFICIENCY IS ASSUMED. 

DETAIL EQUIPMENT: 

MANUAL APPLICABLE  / AVAILABLE

(LOCATION): 


	MACHINE GUARDING AND SHIELDING APPLICABLE

[      ]  YES

[      ]  NO

[      ]  N/A

EMERGENCY STOP / PANIC BUTTON APPLICABLE

[      ]  YES

[      ]  NO

[      ]  N/A

LOCK-OUT TAG APPLICABLE

[      ]  YES

[      ]  NO

[      ]  N/A

OTHER (SUBJECT-SPECIFIC)

[      ]  YES

[      ]  NO

[      ]  N/A
	[     ] Teacher

DATE:

__________

[     ] Board 

DATE: 

__________


	DETAIL STEPS: 

Student attended teacher safety instructions, lessons, demonstration(notes recorded)

Student passed oral or written assessment (test)

Student demonstrated safe setup and operation of equipment to teacher

Student prepared  and delivered power point presentations on all class tools and machines

Student granted permission to use equipment

SIGNAGE: 

safety sign posted 

RESOURCES: 

safety lesson

tool safety video

tool power point presentation

manual

FREQUENCY OF RETRAINING ADVISED: 

Students should be re-trained every semester

Safety passports expire at the end of every semester
	DAILY: 

WEEKLY:

MONTHLY:

ANNUALLY:

CONTACT FOR REPAIR: 





The Ontario Council for Technology Education wishes to acknowledge the contribution of the individuals that participated in the development of the Emphasis Course Resources.
Health and Safety Resources and Curriculum


These resources identify safety rules associated with hazards and processes. They are applicable to a wide range of occupations and situations.


e.g.  Occupational Health and Safety Act, 1990, Live Safe! Work Smart!�


Based on the Ontario curriculum this resource contains safety lessons for technology subjects





Classroom Safety Resources


These resources identify safety policies and procedures that ensure the safety of people in schools (e.g. WHMIS Training Sessions, Board Safety Policies, etc.)


SAFE docs- These resources provide a framework for developing safety procedures in school classrooms�it is highly recommended that all teachers complete an OCTE Safety NET template for their individual experience / program / classroom / school / board. This is an excellent starting point for self-reflection and preparation for MOL/MOE inspection. 





Equipment and Hazard-Specific Safety Rules


These resources are Just-in-Time (JIT) safety rules. They are applicable to specific equipment in the facility and may apply to specific hazards associated with a program emphasis.





These rules are developed at the classroom/school level to implement safe work practices. They may be adapted from a variety of sources including equipment manufacturer’s manuals.  A summary is often posted near equipment.





Safety Management


The teacher develops these resources. The daily classroom safety routines and policies are based on the above safety resources and applied to each individual facility/classroom. Protocols developed to teach safe behavior directly should include managing safe work practices and behavior through demonstration and reinforcement of safe working procedures, establishment of clear safety rules, safety passports, assignments, quizzes, and research.��Again, it is highly recommended that teachers complete a Safety NET template to review their unique projects and procedures and consider risks as advised by OSBIE, and other professional health and safety partners. 








Procurement 


School boards have guidelines and procedures to follow when ordering and installing equipment. It is important to become familiar with and in compliance with these guidelines and procedures.








MANUFACTURING EQUIPMENT SAFETY GUIDELINES CULMINATING ACTIVITY











16
TMY – Welding Emphasis

