Video QR code

(to be inserted)

Thickness Planer

Safety Lesson Plan

| Writer: | Brad Johnson |
| --- | --- |
| Doc version (version Y/M/D date i.e.150630): | 16.02.27 |

| Specific Tool or Process: | Thickness Planer |
| --- | --- |
| Main Applicable Course: | Construction Technology – TCJ2O |
| Additional Applicable Courses: | TCJ3E/3C, TCJ4E/4C, TWJ3E, TWJ4E, Construction Emphasis Courses |

Safety Instruction Type(check all that apply)

| Whole class demo | X | Demo at course start |  |
| --- | --- | --- | --- |
| Small group demo |  | Just in Time (JIT) demo | X |
| Individual demo | X | Assessment Review | X |
| Student presentation |  | Assessment demo | X |
| Students repeat demo | x | Other |  |

This resource was produced
 by the Ontario Council for Technological Education (OCTE)
 It may be used in its entirety, in part, or adapted

Assessment Procedures

Assessment is through quizzes on safe procedures and terminology knowledge. Students must also demonstrate their knowledge and skills through observed operation of the tool. Results of success are signed by the instructor on the student’s Safety Passport as well as (optional) wall charts that allows students to see which equipment they have met qualifications required to use safely

Associated Documents

| Appendices to this lesson | Appendix A: Breaking Out Rough Stock (in this doc) Appendix B: How to Dress Rough Sawn Lumber (in this doc) Appendix C: Student Safety Declaration for Construction Technology Appendix D: Teacher Safety Record (in this doc) Appendix E: Thickness Planer Safety (from OCTE SafeDoc) Appendix F: Thickness Planer Safety Quiz (in this doc) Appendix G: Thickness Planer Terminology Quiz (in this doc) |
| --- | --- |
| OCTE SafeDOC | Construction July 2013 http://octelab.com/filter/%2A?field_lesson_subject_tid%5B%5D=20 |
| OCTE SafetyNet | http://octelab.com/safetynet-pages?keys=&term_node_tid_depth%5B%5D=53&field_id_value= |
| Other Ministry | Curriculum documents - http://www.edu.gov.on.ca/eng/curriculum/secondary/teched910curr09.pdf |
| Other Reference Docs | Live Safe! Work Smart! Grade 9/10 and Grade 11/12 http://www.livesafeworksmart.net Ontario School Boards Insurance Exchange http://www.osbie.on.ca Workplace Safety and Insurance Board http://www.wsib.on.ca Ontario Ministry of Labour Web address: http://www.labour.gov.on.ca/english/ Power Tool Institute - Educational Materials http://powertoolinstitute.com/pti_pages/educational.asp |

Hazard Table:

| Standard Tool Hazards |  | Biological Hazards |  |
| --- | --- | --- | --- |
| Sharp objects, cuts | X | Food contamination |  |
| Knives/sharp tool procedures | X | Food temperature handling |  |
| Set up adjustments, lock procedures | X | Sharps/cutting tools handling |  |
| Machine guarding | X | Exposure to sunlight/ultraviolet light |  |
| High speed moving elements | X | Exposure to blood products |  |
| Pinch points |  | Exposure to mold, mildew |  |
| Sharp noise |  | Temperature extremes |  |
| Constant noise | X | Insect bites/plant/animal hazards |  |
| Drop hazards |  | Emergency contamination proc. |  |
| Kickback hazards | X |  |  |
| Heavy material hazards | X | Chemical Hazards |  |
| Use of recycled products/materials | X | WHMIS identification |  |
| Dust/debris hazards | X | Exposure to dangerous gases/fumes |  |
| Spill hazards |  | Exposure to dangerous liquids |  |
| Emergency stop procedures | X | Exposure to solvents |  |
|  |  | Exposure to pesticides/herbicides |  |
|  |  | Glassware handling |  |
|  |  | Emergency chemical procedures |  |
| Fire/Burn Hazards |  |  |  |
| Flammable material hazards |  | Electrical hazards |  |
| Hot processes |  | Electrical shock |  |
| Sparks |  | Electrical sparks |  |
| Explosion hazards |  | Electrical burns |  |
| Noxious gases/fumes |  | Short circuits |  |
| Emergency fire procedures | X | Wire/cord/cable inspection |  |
|  |  | Electrical connection inspection |  |
|  |  | Power cables/bars handling |  |
| Ergonomic hazards |  | Emergency power shutoff procedures | X |
| Heavy lifting |  |  |  |
| Awkward lifting |  | Site hazards |  |
| Heavy or extreme force |  | Confined spaces |  |
| Constant heavy force |  | Working from heights |  |
| Vibration |  | Slip/trip/fall hazards | X |
| Posture, body placement |  | Hazardous location of others |  |
| Repetitive stress injuries |  |  |  |
| Working with helpers | X |  |  |
| Emergency injury procedures | X |  |  |
| Safe disposal and Cleanup |  | Other hazards (specify) |  |
| Material scrap disposal |  |  |  |
| Chemical disposal |  |  |  |
| Biological materials disposal |  |  |  |
| Tool cleanup procedures | X |  |  |

Personal Protective Equipment

| PPE | Required | Suggested | Details |
| --- | --- | --- | --- |
| Eye, Face, Head Protection |  |  |  |
| Safety glasses | X |  |  |
| Face shields |  |  |  |
| Welding helmets/goggles |  |  |  |
| Hard hats |  |  |  |
| Hearing Protection |  |  |  |
| Ear plugs/ Ear muffs | X |  |  |
| Hand protection |  |  |  |
| Work gloves |  |  |  |
| Disposal gloves |  |  |  |
| Chemical resistant gloves |  |  |  |
| Knife shields |  |  |  |
| Respiration Protection |  |  |  |
| Dust masks | X |  |  |
| Surgical masks |  |  |  |
| Respirator (describe type) |  |  |  |
| Body Protection |  |  |  |
| Foot protection |  |  |  |
| Arm shields |  |  |  |
| Leg shields |  |  |  |
| Bibs | X |  |  |
| Coveralls |  |  |  |
| Welding jackets |  |  |  |
| Hot/Cold temperature protection |  |  |  |
| Fall Arrest |  |  |  |
| Fire protection |  |  |  |

Lesson Preparation Procedures

For this lesson the instructor will need to…

- Prior to the lesson the instructor will check all guards, feeds and adjusters to ensure that they are installed and operating correctly.

- The instructor and students will make sure that the area around the machines to be used will be free of dirt and debris

- Ensure hearing protection and eye protection is provided for everyone in the shop area

- Check push pads and push sticks are safe to use and in the correct area

- Prepare a jointed rough stock board cut to a length of 16” for each student as well as one for demonstration by the instructor.

- Provide each student with a copy of Appendix C: Student Safety Declaration for Construction Technology. This must be signed before students are allowed to use power tools in the shop.

- Appendix D: Teacher Safety Record is your record of student demonstration of safe operation of the jointer, which must be initialled and dated at each stage.

- Only students with returned signed permission forms are allowed into the shop (Appendix C).

Step by Step: Performing the Lesson

- Describe tool/work area inspections, preparations, PPE. Instructor describes the operation of the planer’s knives and automatic feed, and describes the required PPE: safety eyewear, noise protection and dust mask, as well as the rules for loose fitting clothing, shoes tied, and long hair tied back

- Provide a copy of Appendix A Breaking Out Rough Stock and Appendix B: How to Dress Rough Sawn Lumber (FEWTEL) to all students, which is the assignment to be completed once the students have demonstrated safe use of the tools involved with breaking out rough stock

- Make sure students have access to Appendix E: Thickness Planer Safety (from OCTE Safe doc) and go over the safety procedures for using the tool.

- Provide students with a description of the tool and terminology, demonstrating the safe procedures of the tool use and emergency procedures to follow. Ensure all students receive the demonstration, which is recorded in Appendix D: Teacher Safety Record.

- Demonstrate the planer preparation procedures (adjusting, setting, checking, testing, cleaning, checking stops, guards). Demonstrate the location the emergency stop, the start and stop buttons, where the path of kick back lies, the dust collector start and stop buttons, and the minimum length of material that can be run through the planer.

- Students are shown how to inspect the stock for any foreign objects like nails or screws or loose knots.  Teacher discusses the types of emergencies that may occur in using the plan, how to mitigate them and what to do in an emergency.

- Demonstrate how to set the table height to the thickest part of the board, then how to stand and feed the machine and remove the planed wood. Describe how to push with the hips to get stuck stock going, or use of a thin piece of plywood to give it a push if inside the planer, (or if still stuck, how to shut off planer and remove the stock).

- Students run the stock through the planer, inspecting the planed surface, then move the table by 1/8” by the depth adjustment wheel, till desired thickness is reached.

- Students are shown how to wait for the knives to stop spinning and ensure the stop button is engaged before proceeding with the cleanup by brush and/or vacuum.

Emergencies

Before allowing students to perform any procedure, ensure students understand potential hazards and their mitigation.

Students must be trained to be aware of potential hazards, how to deal with emergencies, locations of safety equipment such as fire extinguishers, first aid kits and safety exits. Teachers must ensure safety equipment is up to date and ready.

Any injury, no matter how seemingly insignificant, must be reported to the instructor. Instructors must make reports as per Board and OSBIE requirements.

Fire

- Warn instructor & other students. Stop machine operation and hit shop E-stop as applicable. Instructor must direct students away from the danger and toward fire exit, ensuring fire alarm is activated, and use fire extinguisher if warranted.

- Burns must be treated right away, with water or preferably burn cream. Cover wound and bring student to administration.

- Any injury must be reported as per Board and OSBIE requirements.

Cuts/Scratches/Slicing

- Student must stop operation, tell students close by to watch over equipment, tell instructor, then proceed to wash out the cut/scrape. Disinfectant and bandages should be brought in from the first aid kit and cut dressed.

- Inspect and assess the reason for the cut and inspect the tools and work pieces involved. Clean any blood, skin in the area with disinfectant. Instructors must approve of continuing operation and warn all students about the incident and instruct how to avoid future events.

- Any injuries must be reported as per Board and OSBIE requirements.

Burns

- Burns must be treated right away, with water or preferably burn cream. Cover wound and if warranted, bring student to administration.

- Inspect and assess the reason for the burn, inspect the tools and work pieces involved. Instructors must approve of continuing operation and warn all students about the incident and instruct how to avoid future events.

- Any injury must be reported as per Board and OSBIE requirements.

Lifting strain

- Stop procedure immediately. Safely lower weight.

- Assist person, ensure they cannot fall.

- Assist in moving person to comfortable position or location only when safe to do so.

- Get someone to notify administration immediately.

- Any injury must be reported as per Board and OSBIE requirements.

Electrical Shock

- Students must be trained to notice signs of electrical shock. Person undergoing shock must not be touched until power removed remotely.

- Clear the area; shut off power at fuse panel or e-stop. Unplug equipment only when power is off.

- Get someone to notify administration immediately.

- If shock is severe or prolonged:

  - Begin CPR if the person shows no signs of circulation, such as breathing, coughing or movement.

  - Keep person warm.

- Cover any burned areas with a sterile gauze bandage.

- Leave scene as is for inspectors.

- Any injury must be reported as per Board and OSBIE requirements.

Cleanup Procedures

After the machine has been properly powered down, it is the responsibility of each operator to ensure that the workpiece and any cutoffs are set in a safe location; any debris on the floor and on the machine has been swept up and deposited into proper receptacles; and all push pads, push blocks, and PPE are cleaned and returned to the designated location.

Assessing Safety Knowledge

Students will need to go through the lessons and demonstrate their knowledge in test or quizzes before they will be eligible for a sign off and permission to use any machine.  Following the Ministry of Labour definition of a competent worker, the students will be required to have the knowledge (lesson), skills (student demonstration), and training (teacher demonstration) required before they are eligible to sign off for the safe use of the jointer.  Once the students have been signed off, then they will be able to continue to use the jointer at the teacher’s discretion to complete the breaking out rough stock assignment.

Appendix A: Breaking Out Rough Stock

Mark:             / 28                                                     Name:

Assignment Objective:

- To accurately dress a piece of rough sawn lumber to specified dimensions

- Use machine procedures correctly to dress lumber without any machining errors

Material Needed:

- Rough sawn Pine or Poplar lumber

Tools Needed:

- Compound Mitre Saw

- Jointer

- Table Saw

- Thickness Planer

- Disc & Belt Sander

Specified Dressed Dimensions:

- T = Thickness = ¾ inch

- W = Width = 4 inches

- L = Length = 16 inches

Assignment Assessment Criteria:

| Marking Criteria | Level 1 – 1 Mark | Level 2 – 2 Marks | Level 3 – 3 Marks | Level 4 – 4 Marks |
| --- | --- | --- | --- | --- |
| Thickness | Within 1/8 inch | Within 1/16 inch | Within 1/32 inch | Exact |
| Width | Within 1/8 inch | Within 1/16 inch | Within 1/32 inch | Exact |
| Length | Within 1/8 inch | Within 1/16 inch | Within 1/32 inch | Exact |
| Ends / Edges 90˚ | Most are not 90˚ | Some are 90˚ | Most are 90˚ | All are 90˚ |
| Saw marks | Major error | Clearly visible | Barely visible | None visible |
| Washboard | Major error | Clearly visible | Barely visible | None visible |
| Tearout or Snipe | Major error | Clearly visible | Barely visible | None visible |

| Marking Criteria | Level 1 – 1 Mark | Level 2 – 2 Marks | Level 3 – 3 Marks | Level 4 – 4 Marks |
| --- | --- | --- | --- | --- |
| Thickness |  |  |  |  |
| Width |  |  |  |  |
| Length |  |  |  |  |
| Ends / Edges 90˚ |  |  |  |  |
| Saw marks |  |  |  |  |
| Washboard |  |  |  |  |
| Tearout or Snipe |  |  |  |  |
| Totals |  |  |  |  |

Appendix B: How to Dress Rough Sawn Lumber

Use the F.E.W.T.E.L. process described below:

- FACE - Use the jointer to straighten and flatten one face of the stock.  Place the cupped side down for greater safety.  Always use push blocks or push sticks and never let your bare hands pass over the jointer’s cutter head.

- EDGE- Use the jointer to straighten and flatten one edge of the stock.  Keep the jointed face firmly against the jointer’s fence to create a 90° corner.  If the board is not kept flat against the fence during jointing, the edge and face will not be square (90°) with each other. (Move the stock slower through the jointer if you notice any washboard / ripples on the edge of the stock)

- WIDTH - Use the table saw to rip the board to the width (Add 1/32”) required.  (Place the jointed face down and the jointed edge against the fence)  Now use the jointer to remove the extra 1/32” you added.  Joint the edge with the rough sawmarks left by the table saw’s blade. You now have two perfectly jointed edges.

- THICKNESS- Use the thickness planer to plane the board to the desired thickness.  Place the smooth jointed face down on the table and proceed.  When both faces are smooth you can alternate between the faces being planed and correct any defects while you continue to plane the board to the desired thickness.

- END- Cut one end of the stock square using the compound mitre saw.  Cut slowly to minimize tearout (splintering of the wood) on the back and bottom of the stock.  This is also the time to cut off any end checks (splits) at the board’s end.

- LENGTH - Mark the stock to its final length (add 1/16”) and cut using the compound mitre saw.

# MEASURE TWICE …... CUT ONCE

Appendix C:

Student Safety Declaration for Construction Technology

Personal Protective Equipment [PPE]

1. Avoid wearing loose, baggy clothing or personal accessories, such as watches, chains, rings, or other jewellery, no ties.

2. Wear safety glasses, shields and gloves and other PPE as instructed.

Lift Support and Movement

1. Move a heavy load only with teacher approval.

2. Use assistance to lift items over 20 kilograms ,

3. Secure and support heavy and long loads with approved stands only.

Machine Tools and Energy

1. Do not direct compressed air or gases towards anyone or towards exposed skin or clothing.

2. Operate equipment, tools or machinery only after receiving proper instruction and permission from the teacher. Unless otherwise stated, only one student operator per tool or machine.

3. Never leave equipment or machinery running unattended.

4. Do not attempt to repair any electrical connections.

5. Lockout any equipment which is being repaired.

Storage and Handling of Compressed Gases

1. Complete WHMIS, symbols and recognition instruction.

2. Maintain all cylinders in an upright position, chained and secured.

3. Only teachers may change gas bottles.

Storage and Handling of Chemical Substances

1. Understand and follow WHMIS, and MSDS instruction before handling chemical substances.

2. Secure all flammable and corrosives in approved cabinets.

3. Maintain good housekeeping practices when dealing with chemical substances.

Waste Disposal and Recycling

1. Be responsible for cleaning up workstations, tools and the shops.

2. Sort waste by category as required using approved containers.

3. Sort recyclable liquids and solids into proper approved storage container.

Ensure a Safe Workplace

1. Inform teachers of all injuries, damaged equipment and potentially dangerous situations.

2. Know where all fire exits and power shutdown switches are and how to use them during emergency situations.

3. Do not compromise the safety of others through horseplay or aggressive action.

4. Only use equipment when properly trained, always with any necessary personal protective equipment, and when all related safety issues are understood.

5. Ask for assistance from the teacher when unsure of the proper procedures or health and safety issues.

Health Issues, Prescribed and Non-prescribed Medications

1. Report any potential health issues to the teacher at the beginning of class.

2. Report any use of prescription medications and will inform teachers of any possible side effects of the medication [e.g. penicillin, phenobarbital etc.].

3. Report any use of non-prescription medication and any possible side effects of the medication [e.g. Reactine, Benadril, any cough syrups etc.].

4. Never enter a shop or lab carrying, or under the influence of illegal substances.

Medication, Health or Other Concerns

Please indicate any medication, health, or other concerns that may impact the student’s judgment or behaviour in the classroom:

Consequences for Improper Action

I,				 have read and understood the safety rules for this Technological Education class. I understand that failure to comply with this agreement may result in injury to myself or others. I am aware that others may be added concerning specific tools, machines and operations as they are introduced as part of the course. I agree to abide by the safety rules for my own safety and the safety of those around me. I understand that my failure to observe the safety procedures will result in my privileges being withdrawn.

Student name (print): Student signature:

____________________________________________

Parent/guardian name if under 18 (print):

____________________________________________

Parent/guardian signature:

________________________________________________________

Date: ___________________________________________________

| Appendix D: Teacher Safety Record – By Focus Teacher Safety Record For: Thickness Planer The teacher will ensure through a lesson, test and demonstration that students understand the safety procedures that are required and the correct PPE to be worn while using the Jointer. Before students are able to use the jointer the teacher and student must initial all areas of the Safety Record. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Student Name |  | Attended Teacher Instruction/Demo |  | Passed Written/Oral Testing |  | Demonstrated Safe Use/technique |  |
| Date | Student Initial |  | Date | Student Initial | Date |  | Student Initial |
| 1. |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |
| 9. |  |  |  |  |  |  |  |

# Appendix E: # Thickness Planer

- Never plane boards shorter than (Teacher - see your owner’s manual for minimum sizes and enter them here) __ inches or thinner than ____ inch. - Do not plane two boards at the same time. - Be sure your wood is free of foreign materials, nails, grit, dirt, loose knots or anything else that seems out of place. - Before investigating any jammed pieces shut the power off and wait until the blades have come to a complete stop. - Never reach into the planer. Use a thin push stick when necessary to help push your wood through. - Keep fingers from under the stock while feeding or retrieving. This will prevent fingers from being pinched between the machine and board. - Stand clear of possible kickback – stand slightly to the side. - This is the only machine where is to okay to have a partner help. One person feeds the machine, while a second unloads the wood. - Make successive passes removing shallow amounts. Don’t try and take too much off at once (Teacher - see your owner’s manual for maximum amount that can be removed in one pass sizes and enter it here). AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR

# Appendix F: # THICKNESS PLANER QUIZ

Name:_______________________________ Date:________________

1. Check the floor for any __________________. 2. Do not plane ____________ boards at the same time. 3. Do not take off too much at once, max _____ for each pass. 4. For most planning, you start the machine at the _______ ______ setting. 5. Use a ___________ stick when necessary. 7. Make sure you put on your ___________ _____________ before starting the machine.. 8. Assume a position with your body _________________ of a possible kickback. 9. Remove all _________________ and tie back _________________ hair. 10. Before investigating any jammed pieces _____________ the power off and ______________ till the cutters have stopped spinning

WORD BANK: Hazards, clear, wait, one inch, two, push safety glasses, shut, long, jewellery (Teacher – add the max that can be removed in one pass)

Appendix G: Thickness Planer Terminology Quiz