Video QR code

(to be inserted)

Jointer

Safety Lesson Plan

| Writer: | Brad Johnson |
| --- | --- |
| Doc version (version Y/M/D date i.e.150630): | 16.02.27 |

| Specific Tool or Process: | Jointer |
| --- | --- |
| Main Applicable Course: | Construction Technology – TCJ2O |
| Additional Applicable Courses: | TCJ3E/3C, TCJ4E/4C, TWJ3E, TWJ4E, Construction Emphasis Courses |

Safety Instruction Type(check all that apply)

| Whole class demo | X | Demo at course start |  |
| --- | --- | --- | --- |
| Small group demo |  | Just in Time (JIT) demo | X |
| Individual demo | X | Assessment Review | X |
| Student presentation |  | Assessment demo | X |
| Students repeat demo | x | Other |  |

This resource was produced
 by the Ontario Council for Technological Education (OCTE)
 It may be used in its entirety, in part, or adapted

Assessment Procedures

Assessment is through quizzes on safe procedures and terminology knowledge. Students must also demonstrate their knowledge and skills through observed operation of the tool. Results of success are signed by the instructor on the student’s Safety Passport as well as (optional) wall charts that allows students to see which equipment they have met qualifications required to use safely

Associated Documents

| Appendices to this lesson | Appendix A: Breaking Out Rough Stock (in this doc) Appendix B: How to Dress Rough Sawn Lumber (in this doc) Appendix C: Student Safety Declaration for Construction Technology Appendix D: Teacher Safety Record (in this doc) Appendix E: Wood Jointer Safety (from OCTE SafeDoc) Appendix F: Jointer Safety Quiz (in this doc) Appendix G: Jointer Terminology Quiz (in this doc) |
| --- | --- |
| OCTE SafeDOC | Construction July 2013 http://octelab.com/filter/%2A?field_lesson_subject_tid%5B%5D=20 |
| OCTE SafetyNet | http://octelab.com/safetynet-pages?keys=&term_node_tid_depth%5B%5D=53&field_id_value= |
| Other Ministry | Curriculum documents - http://www.edu.gov.on.ca/eng/curriculum/secondary/teched910curr09.pdf |
| Other Reference Docs | Live Safe! Work Smart! Grade 9/10 and Grade 11/12 http://www.livesafeworksmart.net Ontario School Boards Insurance Exchange http://www.osbie.on.ca Workplace Safety and Insurance Board http://www.wsib.on.ca Ontario Ministry of Labour Web address: http://www.labour.gov.on.ca/english/ |

Hazard Table:

| Standard Tool Hazards |  | Biological Hazards |  |
| --- | --- | --- | --- |
| Sharp objects, cuts | X | Food contamination |  |
| Knives/sharp tool procedures | X | Food temperature handling |  |
| Set up adjustments, lock procedures | X | Sharps/cutting tools handling |  |
| Machine guarding | X | Exposure to sunlight/ultraviolet light |  |
| High speed moving elements | X | Exposure to blood products |  |
| Pinch points |  | Exposure to mold, mildew |  |
| Sharp noise |  | Temperature extremes |  |
| Constant noise | X | Insect bites/plant/animal hazards |  |
| Drop hazards |  | Emergency contamination proc. |  |
| Kickback hazards | X |  |  |
| Heavy material hazards | X | Chemical Hazards |  |
| Use of recycled products/materials | X | WHMIS identification |  |
| Dust/debris hazards | X | Exposure to dangerous gases/fumes |  |
| Spill hazards |  | Exposure to dangerous liquids |  |
| Emergency stop procedures | X | Exposure to solvents |  |
|  |  | Exposure to pesticides/herbicides |  |
|  |  | Glassware handling |  |
|  |  | Emergency chemical procedures |  |
| Fire/Burn Hazards |  |  |  |
| Flammable material hazards |  | Electrical hazards |  |
| Hot processes |  | Electrical shock |  |
| Sparks |  | Electrical sparks |  |
| Explosion hazards |  | Electrical burns |  |
| Noxious gases/fumes |  | Short circuits |  |
| Emergency fire procedures | X | Wire/cord/cable inspection |  |
|  |  | Electrical connection inspection |  |
|  |  | Power cables/bars handling |  |
| Ergonomic hazards |  | Emergency power shutoff procedures | X |
| Heavy lifting |  |  |  |
| Awkward lifting |  | Site hazards |  |
| Heavy or extreme force |  | Confined spaces |  |
| Constant heavy force |  | Working from heights |  |
| Vibration |  | Slip/trip/fall hazards | X |
| Posture, body placement |  | Hazardous location of others |  |
| Repetitive stress injuries |  |  |  |
| Working with helpers | X |  |  |
| Emergency injury procedures | X |  |  |
| Safe disposal and Cleanup |  | Other hazards (specify) |  |
| Material scrap disposal |  |  |  |
| Chemical disposal |  |  |  |
| Biological materials disposal |  |  |  |
| Tool cleanup procedures | X |  |  |

Personal Protective Equipment

| PPE | Required | Suggested | Details |
| --- | --- | --- | --- |
| Eye, Face, Head Protection |  |  |  |
| Safety glasses | X |  |  |
| Face shields |  |  |  |
| Welding helmets/goggles |  |  |  |
| Hard hats |  |  |  |
| Hearing Protection |  |  |  |
| Ear plugs/ Ear muffs | X |  |  |
| Hand protection |  |  |  |
| Work gloves |  |  |  |
| Disposal gloves |  |  |  |
| Chemical resistant gloves |  |  |  |
| Knife shields |  |  |  |
| Respiration Protection |  |  |  |
| Dust masks | X |  |  |
| Surgical masks |  |  |  |
| Respirator (describe type) |  |  |  |
| Body Protection |  |  |  |
| Foot protection |  |  |  |
| Arm shields |  |  |  |
| Leg shields |  |  |  |
| Bibs | X |  |  |
| Coveralls |  |  |  |
| Welding jackets |  |  |  |
| Hot/Cold temperature protection |  |  |  |
| Fall Arrest |  |  |  |
| Fire protection |  |  |  |

Lesson Preparation Procedures

For this lesson the instructor will need to…

- Prior to the lesson the instructor will check all guards to ensure that they are installed and operating correctly. If the Jointer is not hardwired, all electrical connections/wires need to be inspected.

- The instructor and students will make sure that the area around the machines to be used will be free of dirt and debris

- Ensure hearing protection and eye protection is provided for everyone in the shop area

- Check push pads and push sticks are safe to use and in the correct area

- Prepare a rough stock board cut to a length of 16” for each student as well as one for demonstration by the instructor.

- Provide each student with a copy of Appendix C: Student Safety Declaration for Construction Technology. This must be signed before students are allowed to use power tools in the shop.

- Appendix D: Teacher Safety Record is your record of student demonstration of safe operation of the jointer, which must be initialled and dated at each stage.

- Only students with returned signed permission forms are allowed into the shop (Appendix D).

Step by Step: Performing the Lesson

- Describe tool/work area inspections, preparations, and the required PPE: safety eyewear, noise protection and dust mask, as well as the rules for loose fitting clothing, shoes tied, and long hair tied back.

- Provide a copy of Appendix A Breaking Out Rough Stock and Appendix B: How to Dress Rough Sawn Lumber (FEWTEL) to all students, which is the assignment to be completed once the students have demonstrated safe use of the tools involved with breaking out rough stock

- Make sure students have access to Appendix E: Wood Jointer Safety (from OCTE Safe doc) and go over the safety procedures for using the tool.

- Provide students with a description of the tool and terminology, demonstrating the safe procedures of the tool use and emergency procedures to follow. Ensure all students receive the demonstration, which is recorded in Appendix D: Teacher Safety Record.

- Demonstrate the jointer tool preparation procedures (adjusting, setting, locking down, checking, testing, cleaning, checking stops, guards)

- Demonstrate how to inspect and test that the table adjusts correctly, and the depth indicator scale is accurate

- Next demonstrate the procedure to inspect and prepare the workpiece (cleaning the stock, inspection for loose knots, nails, etc., checking for waft, weave and warp)

- Demonstrate where to stand, how the buddy system will work, and where the emergency stops and power stops are located.

- Discuss the types of emergencies that may occur in using the jointer, how to mitigate them and what to do in an emergency.

- Students are asked to complete the two Quizzes, Appendix F: Jointer Safety Quiz (in this doc) and Appendix G: Jointer Terminology Quiz (in this doc)

- Once students have been successful on the quizzes, the students perform dressing the provided lumber, demonstrating to the instructor the safe use of the jointer.

- Students are shown how to wait for the knives to stop spinning and ensure the stop button is engaged before proceeding with the cleanup by brush and/or vacuum.

Emergencies

Before allowing students to perform any procedure, ensure students understand potential hazards and their mitigation.

Students must be trained to be aware of potential hazards, how to deal with emergencies, locations of safety equipment such as fire extinguishers, first aid kits and safety exits. Teachers must ensure safety equipment is up to date and ready.

Any injury, no matter how seemingly insignificant, must be reported to the instructor. Instructors must make reports as per Board and OSBIE requirements.

Fire

- Warn instructor & other students. Stop machine operation and hit shop E-stop as applicable. Instructor must direct students away from the danger and toward fire exit, ensuring fire alarm is activated, and use fire extinguisher if warranted.

- Burns must be treated right away, with water or preferably burn cream. Cover wound and bring student to administration.

- Any injury must be reported as per Board and OSBIE requirements.

Cuts/Scratches/Slicing

- Student must stop operation, tell students close by to watch over equipment, tell instructor, then proceed to wash out the cut/scrape. Disinfectant and bandages should be brought in from the first aid kit and cut dressed.

- Inspect and assess the reason for the cut and inspect the tools and work pieces involved. Clean any blood, skin in the area with disinfectant. Instructors must approve of continuing operation and warn all students about the incident and instruct how to avoid future events.

- Any injuries must be reported as per Board and OSBIE requirements.

Burns

- Burns must be treated right away, with water or preferably burn cream. Cover wound and if warranted, bring student to administration.

- Inspect and assess the reason for the burn, inspect the tools and work pieces involved. Instructors must approve of continuing operation and warn all students about the incident and instruct how to avoid future events.

- Any injury must be reported as per Board and OSBIE requirements.

Lifting strain

- Stop procedure immediately. Safely lower weight.

- Assist person, ensure they cannot fall.

- Assist in moving person to comfortable position or location only when safe to do so.

- Get someone to notify administration immediately.

- Any injury must be reported as per Board and OSBIE requirements.

Electrical Shock

- Students must be trained to notice signs of electrical shock. Person undergoing shock must not be touched until power removed remotely.

- Clear the area; shut off power at fuse panel or e-stop. Unplug equipment only when power is off.

- Get someone to notify administration immediately.

- If shock is severe or prolonged:

  - Begin CPR if the person shows no signs of circulation, such as breathing, coughing or movement.

  - Keep person warm.

- Cover any burned areas with a sterile gauze bandage.

- Leave scene as is for inspectors.

- Any injury must be reported as per Board and OSBIE requirements.

Cleanup Procedures

After the machine has been properly powered down, it is the responsibility of each operator to ensure that the workpiece and any cutoffs are set in a safe location; any debris on the floor and on the machine has been swept up and deposited into proper receptacles; and all push pads, push blocks, and PPE are cleaned and returned to the designated location.

Assessing Safety Knowledge

Students will need to go through the lessons and demonstrate their knowledge in test or quizzes before they will be eligible for a sign off and permission to use any machine.  Following the Ministry of Labour definition of a competent worker, the students will be required to have the knowledge (lesson), skills (student demonstration), and training (teacher demonstration) required before they are eligible to sign off for the safe use of the jointer.  Once the students have been signed off, then they will be able to continue to use the jointer at the teacher’s discretion to complete the breaking out rough stock assignment.

Appendix A: Breaking Out Rough Stock

Mark:             / 28                                                     Name:

Assignment Objective:

- To accurately dress a piece of rough sawn lumber to specified dimensions

- Use machine procedures correctly to dress lumber without any machining errors

Material Needed:

- Rough sawn Pine or Poplar lumber

Tools Needed:

- Compound Mitre Saw

- Jointer

- Table Saw

- Thickness Planer

- Disc & Belt Sander

Specified Dressed Dimensions:

- T = Thickness = ¾ inch

- W = Width = 4 inches

- L = Length = 16 inches

Cont…

Assignment Assessment Criteria:

| Marking Criteria | Level 1 – 1 Mark | Level 2 – 2 Marks | Level 3 – 3 Marks | Level 4 – 4 Marks |
| --- | --- | --- | --- | --- |
| Thickness | Within 1/8 inch | Within 1/16 inch | Within 1/32 inch | Exact |
| Width | Within 1/8 inch | Within 1/16 inch | Within 1/32 inch | Exact |
| Length | Within 1/8 inch | Within 1/16 inch | Within 1/32 inch | Exact |
| Ends / Edges 90˚ | Most are not 90˚ | Some are 90˚ | Most are 90˚ | All are 90˚ |
| Saw marks | Major error | Clearly visible | Barely visible | None visible |
| Washboard | Major error | Clearly visible | Barely visible | None visible |
| Tearout or Snipe | Major error | Clearly visible | Barely visible | None visible |

| Marking Criteria | Level 1 – 1 Mark | Level 2 – 2 Marks | Level 3 – 3 Marks | Level 4 – 4 Marks |
| --- | --- | --- | --- | --- |
| Thickness |  |  |  |  |
| Width |  |  |  |  |
| Length |  |  |  |  |
| Ends / Edges 90˚ |  |  |  |  |
| Saw marks |  |  |  |  |
| Washboard |  |  |  |  |
| Tearout or Snipe |  |  |  |  |
| Totals |  |  |  |  |

Appendix B: How to Dress Rough Sawn Lumber

Use the F.E.W.T.E.L. process described below:

- FACE - Use the jointer to straighten and flatten one face of the stock.  Place the cupped side down for greater safety.  Always use push blocks or push sticks and never let your bare hands pass over the jointer’s cutter head.

- EDGE- Use the jointer to straighten and flatten one edge of the stock.  Keep the jointed face firmly against the jointer’s fence to create a 90° corner.  If the board is not kept flat against the fence during jointing, the edge and face will not be square (90°) with each other. (Move the stock slower through the jointer if you notice any washboard / ripples on the edge of the stock)

- WIDTH - Use the table saw to rip the board to the width (Add 1/32”) required.  (Place the jointed face down and the jointed edge against the fence)  Now use the jointer to remove the extra 1/32” you added.  Joint the edge with the rough sawmarks left by the table saw’s blade. You now have two perfectly jointed edges.

- THICKNESS- Use the thickness planer to plane the board to the desired thickness.  Place the smooth jointed face down on the table and proceed.  When both faces are smooth you can alternate between the faces being planed and correct any defects while you continue to plane the board to the desired thickness.

- END- Cut one end of the stock square using the compound mitre saw.  Cut slowly to minimize tearout (splintering of the wood) on the back and bottom of the stock.  This is also the time to cut off any end checks (splits) at the board’s end.

- LENGTH - Mark the stock to its final length (add 1/16”) and cut using the compound mitre saw.

# MEASURE TWICE …... CUT ONCE

Appendix C:

Student Safety Declaration for Construction Technology

Personal Protective Equipment [PPE]

1. Avoid wearing loose, baggy clothing or personal accessories, such as watches, chains, rings, or other jewellery, no ties.

2. Wear safety glasses, shields and gloves and other PPE as instructed.

Lift Support and Movement

1. Move a heavy load only with teacher approval.

2. Use assistance to lift items over 20 kilograms ,

3. Secure and support heavy and long loads with approved stands only.

Machine Tools and Energy

1. Do not direct compressed air or gases towards anyone or towards exposed skin or clothing.

2. Operate equipment, tools or machinery only after receiving proper instruction and permission from the teacher. Unless otherwise stated, only one student operator per tool or machine.

3. Never leave equipment or machinery running unattended.

4. Do not attempt to repair any electrical connections.

5. Lockout any equipment which is being repaired.

Storage and Handling of Compressed Gases

1. Complete WHMIS, symbols and recognition instruction.

2. Maintain all cylinders in an upright position, chained and secured.

3. Only teachers may change gas bottles.

Storage and Handling of Chemical Substances

1. Understand and follow WHMIS, and MSDS instruction before handling chemical substances.

2. Secure all flammable and corrosives in approved cabinets.

3. Maintain good housekeeping practices when dealing with chemical substances.

Waste Disposal and Recycling

1. Be responsible for cleaning up workstations, tools and the shops.

2. Sort waste by category as required using approved containers.

3. Sort recyclable liquids and solids into proper approved storage container.

Ensure a Safe Workplace

1. Inform teachers of all injuries, damaged equipment and potentially dangerous situations.

2. Know where all fire exits and power shutdown switches are and how to use them during emergency situations.

3. Do not compromise the safety of others through horseplay or aggressive action.

4. Only use equipment when properly trained, always with any necessary personal protective equipment, and when all related safety issues are understood.

5. Ask for assistance from the teacher when unsure of the proper procedures or health and safety issues.

Health Issues, Prescribed and Non-prescribed Medications

1. Report any potential health issues to the teacher at the beginning of class.

2. Report any use of prescription medications and will inform teachers of any possible side effects of the medication [e.g. penicillin, phenobarbital etc.].

3. Report any use of non-prescription medication and any possible side effects of the medication [e.g. Reactine, Benadril, any cough syrups etc.].

4. Never enter a shop or lab carrying, or under the influence of illegal substances.

Medication, Health or Other Concerns

Please indicate any medication, health, or other concerns that may impact the student’s judgment or behaviour in the classroom:

Consequences for Improper Action

I,				 have read and understood the safety rules for this Technological Education class. I understand that failure to comply with this agreement may result in injury to myself or others. I am aware that others may be added concerning specific tools, machines and operations as they are introduced as part of the course. I agree to abide by the safety rules for my own safety and the safety of those around me. I understand that my failure to observe the safety procedures will result in my privileges being withdrawn.

Student name (print): Student signature:

____________________________________________

Parent/guardian name if under 18 (print):

____________________________________________

Parent/guardian signature:

________________________________________________________

Date: ___________________________________________________

| Appendix D: Teacher Safety Record – By Focus Teacher Safety Record For: Jointer The teacher will ensure through a lesson, test and demonstration that students understand the safety procedures that are required and the correct PPE to be worn while using the Jointer. Before students are able to use the jointer the teacher and student must initial all areas of the Safety Record. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Student Name |  | Attended Teacher Instruction/Demo |  | Passed Written/Oral Testing |  | Demonstrated Safe Use/technique |  |
| Date | Student Initial |  | Date | Student Initial | Date |  | Student Initial |
| 1. |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |
| 9. |  |  |  |  |  |  |  |

# Appendix E: # Wood Jointer Safety

- Check the board for foreign objects – such as nails or rocks. - Never joint stock containing loose or unsound knots. - Make sure stock is at least 12” long and ½” thick. - Ensure the guard is installed and working properly. - Do not change depth of cut without permission from the teacher. Depth of cut should be set to 1/16th or less. - Always use a push stick and/or push block. Never let a finger come with in 10 cm (4") of blade when running. - Never make "free hand" cuts on the jointer. Always use the fence. Ask for assistance when working with large pieces. - Always push the work well beyond the blade when finishing a cut so the guard closes back over the blade. - Never let go of your wood part way through a cut. - If the board jams while jointing, *****DO NOT EVER *****pull back on the front of the board. - Before investigating any jammed pieces shut the power off and lock it out. - Turn the jointer off immediately if it does not sound right or if slivers of wood catch between the blade and table. AT ALL TIMES – IF IN DOUBT, SEE YOUR INSTRUCTOR

# Appendix F: # JOINTER QUIZ

Name:_______________________________ Date:________________

1. Always wear _______________ glasses when using this machine. 2. Never ___________ part way through a cut. 3. Check the ____________to make sure there are no nails, stones, or loose knots. 4. Check the ____________________. Make sure it returns to cover the blades. 5. Always use a ________ block or push ___________ . 6. Get ______________________ when working on large pieces . 7. Always push your wood _____________ the guard so that it can return and _____________ the blades. 8. Stand _______________ of the path of a kickback. 9. Never joint boards shorter than ________________ or thinner than _________.

Appendix G: Jointer Terminology Quiz