3D Modelling Project: Wooden Toy Vehicle

Research Assignment

Complete the following for each of the existing toy vehicles you examine.

Source: (book, magazine, or website)

Type of Vehicle:

Overall dimensions:(LxWxH)

Number of unique parts:

Attach the plans, illustrations, photos, and bill of materials for this vehicle

In the table on the next page, list the parts for the vehicle and assess the difficulty of modelling each piece using the guide below

1-very simple

- an extrusion of a primary geometric shape such as a circle or rectangle; part could be made in one step

2-easy

- An extrusion with simple geometry (for example an L-shape) that could be made in one step; simple geometry with a cut or addition that could be made in 2 steps; a revolution with a simple profile

3-moderate

- An extruded or revolved part with complicated geometry that requires numerous dimensions to define it; a part requiring multiple steps to make it, possibly including extrusions, revolutions, fillets, or chamfers; a lofted part with simple geometry

4-difficult

- A part requiring multiple steps to make it; complex geometry requiring many dimensions to define it; a part requiring work planes; a part requiring tools such as mirror and array; a lofted part with complex geometry requiring more than 2 sketches; a revolved part with complex geometry

| Name of Part | Quantity Req’d | Dimensions | Level of Difficulty | Steps you would take to model this part |
| --- | --- | --- | --- | --- |