Teacher:							Name: __________________

Quiz to confirm knowledge…. 	Assessment before building of relay circuit.           /7

| Which colour wire end connector is used with 14-16 gauge (1.0-2.0 mm2) wire? | *Blue |
| --- | --- |
|  | Red |
|  | Yellow |
|  | Black |
| Which of the following scenarios would cause personal all injury while working with relay and their circuitry? | A spark from a car battery could cause an explosion. |
|  | Using a knife instead of wire strippers to strip wire. |
|  | Not using proper tool to assemble the relay and other components to the electrical board. |
|  | Not understanding that a load and a fuse need to be in an active circuit or burns from excessive heat in wiring may occur. |
|  | *All of the above |
| How many circuits are in a relay? | 1 |
|  | *2 |
|  | 3 |
|  | 4 |
| Which relay circuit has the switch located in it? | The circuit with the contact points. |
|  | *The circuit with the electromagnetic coil. |
|  | The circuit that contains the load. |
| Using a voltmeter (and having an automotive 12 volt battery connected to the circuit), what would you expect the voltage to be before the load of a circuit that has been turned on? | _______________________volts |
|  | 0 |
|  | 6 |
|  | *12 |
|  | 24 |
| Which circuit should be protected by a fuse? | The circuit with the load |
|  | The circuit with the electromagnetic coil |
|  | *Both should be protected by a fuse |
| True or False? The purpose of one of the circuits in a relay is to control another circuit? | *True or False |