Sample DHT Temperature/Humidity Arduino Sketch

https://github.com/futureshocked/arduino_sbs/blob/master/Temperature%20and%20Humidity/DHTtester/DHTtester.ino

Adafruit Github DHT library files

https://github.com/adafruit/DHT-sensor-library

Adafruit DHT tutorial

https://learn.adafruit.com/dht

// Example testing sketch for various DHT humidity/temperature sensors

// Written by ladyada, slightly modified, public domain

#include "DHT.h"

#define DHTPIN 2 // what pin we're connected to

// Uncomment whatever type you're using!

//#define DHTTYPE DHT11 // DHT 11

#define DHTTYPE DHT22 // DHT 22 (AM2302)

//#define DHTTYPE DHT21 // DHT 21 (AM2301)

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

Serial.println("DHT test");

dht.begin();

}

void loop() {

// Reading temperature or humidity takes about 250 milliseconds!

// Sensor readings may also be up to 2 seconds 'old' (its a very slow sensor)

float h = dht.readHumidity();

float t = dht.readTemperature();

// check if returns are valid, if they are NaN (not a number) then something went wrong!

if (isnan(t) ¦¦ isnan(h)) {

Serial.println("Failed to read from DHT");

} else {

// Uncomment these lines if you prefer to use the Fahrenheit scale

// instead of Celsius. Remember to change line 44 so that the

// symbol is "F" instead of "C"

// float fahrenheitTemp = t * 9.0/5.0+32.0;

// Serial.print("Temperature: ");

// Serial.print(fahrenheitTemp);

// Serial.println(" F");

Serial.print("Humidity: ");

Serial.print(h);

Serial.print(" %\t");

Serial.print("Temperature: ");

Serial.print(t);

Serial.println(" *C");

}

delay(5000); //Insert a 5sec delay between readings

}