Arduino Data Types Reference Sheet

Data can take many forms that must be declared properly for later mathematical analysis. In data collection, you assign a variable to contain the data with the proper data type to store incoming data and analyze later.

| Data Type Name | Use | Range | Programming form |
| --- | --- | --- | --- |
| Boolean | Yes/No, On/Off, High/Low, 5V/0V | +5V or 0V (ground) | HIGH, LOW |
| char | Single character (also used as ASCII keyboard value for calc (eg A=65) | ASCII alphabet, upper/lower case | ‘A’ |
| Unsigned char or byte | Number from 0 to 255 | 0 - +255 | unsigned char myVar = 200 byte=B100101 B=binary) |
| int (integer) | Counters, variables | -32,768 to 32,767 (16bit/2byte) ATMega boards, others 4 byte | Int myVar = 23 |
| unsigned int | Counters, variables | 0 – 65535 (ATMega boards) | unsigned int ledPin = 13 |
| word | Counters, variables | 16 bit (ATMega boards) | word w = 10000 |
| long | Variables | -2,147,483,648 to 2,147,483,647 (16bit) | long myVar = 18000 (Note: in calc, one long var at least must end in ‘L’ to force it) |
| unsigned long | variables | 0 to +4,294,967,295 (32 bits) | unsigned long myVar = 12657 |
| float | Decimal variables | 3.4028235E+38 to -3.4028235E+38 (4 byte) | float myVar = -327.976 |
| double | Decimal variables | Same as float except Arduino Due: 8 byte precision | double myVAr = 1234.5678 |
| string | Text | Array of char types | Str1[9] (9 characters) |
| array | Array of char, can be indexed | Array of char types (1st index is 0) | int myPins[] = {2, 4, 8, 3, 6}; |
| enum | User defined array of constants | Any number of constants | enum myVar { unknown, true, false }; |
| struct | User defined that can have different data types | Any number of data | struct RGB { byte r; byte g; byte b; }; |
| pointer | Variable to point to other in memory | Any variable | var1 -> var2 |