Appendix 12

Arduino Data Logger Sample Code

Note: This code adapted from

https://learn.adafruit.com/adafruit-data-logger-shield/using-the-real-time-clock-2

Modify to suit your own needs. Fahrenheit temperature conversion left out here.

*********************************************************************

#include <SPI.h>

#include <SD.h>

#include <Wire.h>

#include "RTClib.h"

// A simple data logger for the Arduino analog pins

// how many milliseconds between grabbing data and logging it. 1000 ms is once a second

#define LOG_INTERVAL  1000 // mills between entries (reduce to take more/faster data)

// how many milliseconds before writing the logged data permanently to disk

// set it to the LOG_INTERVAL to write each time (safest)

// set it to 10*LOG_INTERVAL to write all data every 10 datareads, you could lose up to

// the last 10 reads if power is lost but it uses less power and is much faster!

#define SYNC_INTERVAL 1000 // mills between calls to flush() - to write data to the card

uint32_t syncTime = 0; // time of last sync()

#define ECHO_TO_SERIAL   1 // echo data to serial port

#define WAIT_TO_START    0 // Wait for serial input in setup()

// the digital pins that connect to the LEDs

#define redLEDpin 2

#define greenLEDpin 3

// The analog pins that connect to the sensors

#define photocellPin 0           // analog 0

#define tempPin 1                // analog 1

#define BANDGAPREF 14            // special indicator that we want to measure the bandgap

#define aref_voltage 3.3         // we tie 3.3V to ARef and measure it with a multimeter!

#define bandgap_voltage 1.1      // this is not super guaranteed but its not -too- off

RTC_DS1307 RTC; // define the Real Time Clock object

// for the data logging shield, we use digital pin 10 for the SD cs line

const int chipSelect = 10;

// the logging file

File logfile;

void error(char *str)

{

Serial.print("error: ");

Serial.println(str);

// red LED indicates error

digitalWrite(redLEDpin, HIGH);

while(1);

}

void setup(void)

{

Serial.begin(9600);

Serial.println();

// use debugging LEDs

pinMode(redLEDpin, OUTPUT);

pinMode(greenLEDpin, OUTPUT);

#if WAIT_TO_START

Serial.println("Type any character to start");

while (!Serial.available());

#endif //WAIT_TO_START

// initialize the SD card

Serial.print("Initializing SD card...");

// make sure that the default chip select pin is set to

// output, even if you don't use it:

pinMode(10, OUTPUT);

// see if the card is present and can be initialized:

if (!SD.begin(chipSelect)) {

error("Card failed, or not present");

}

Serial.println("card initialized.");

// create a new file

char filename[] = "LOGGER00.CSV";

for (uint8_t i = 0; i < 100; i++) {

filename[6] = i/10 + '0';

filename[7] = i%10 + '0';

if (! SD.exists(filename)) {

// only open a new file if it doesn't exist

logfile = SD.open(filename, FILE_WRITE);

break;  // leave the loop!

}

}

if (! logfile) {

error("couldnt create file");

}

Serial.print("Logging to: ");

Serial.println(filename);

// connect to RTC

Wire.begin();

if (!RTC.begin()) {

logfile.println("RTC failed");

#if ECHO_TO_SERIAL

Serial.println("RTC failed");

#endif  //ECHO_TO_SERIAL

}

logfile.println("millis,stamp,datetime,light,temp,vcc");

#if ECHO_TO_SERIAL

Serial.println("millis,stamp,datetime,light,temp,vcc");

#endif //ECHO_TO_SERIAL

// If you want to set the aref to something other than 5v

analogReference(EXTERNAL);

}

void loop(void)

{

DateTime now;

// delay for the amount of time we want between readings

delay((LOG_INTERVAL -1) - (millis() % LOG_INTERVAL));

digitalWrite(greenLEDpin, HIGH);

// log milliseconds since starting

uint32_t m = millis();

logfile.print(m);           // milliseconds since start

logfile.print(", ");

#if ECHO_TO_SERIAL

Serial.print(m);         // milliseconds since start

Serial.print(", ");

#endif

// fetch the time

now = RTC.now();

// log time

logfile.print(now.unixtime()); // seconds since 1/1/1970

logfile.print(", ");

logfile.print('"');

logfile.print(now.year(), DEC);

logfile.print("/");

logfile.print(now.month(), DEC);

logfile.print("/");

logfile.print(now.day(), DEC);

logfile.print(" ");

logfile.print(now.hour(), DEC);

logfile.print(":");

logfile.print(now.minute(), DEC);

logfile.print(":");

logfile.print(now.second(), DEC);

logfile.print('"');

#if ECHO_TO_SERIAL

Serial.print(now.unixtime()); // seconds since 1/1/1970

Serial.print(", ");

Serial.print('"');

Serial.print(now.year(), DEC);

Serial.print("/");

Serial.print(now.month(), DEC);

Serial.print("/");

Serial.print(now.day(), DEC);

Serial.print(" ");

Serial.print(now.hour(), DEC);

Serial.print(":");

Serial.print(now.minute(), DEC);

Serial.print(":");

Serial.print(now.second(), DEC);

Serial.print('"');

#endif //ECHO_TO_SERIAL

analogRead(photocellPin);

delay(10);

int photocellReading = analogRead(photocellPin);

analogRead(tempPin);

delay(10);

int tempReading = analogRead(tempPin);

// converting that reading to voltage, for 3.3v arduino use 3.3, for 5.0, use 5.0

float voltage = tempReading * aref_voltage / 1024;

float temperatureC = (voltage - 0.5) * 100 ;

logfile.print(", ");

logfile.print(photocellReading);

logfile.print(", ");

logfile.print(temperatureC);

#if ECHO_TO_SERIAL

Serial.print(", ");

Serial.print(photocellReading);

Serial.print(", ");

Serial.print(temperatureC);

#endif //ECHO_TO_SERIAL

// Log the estimated 'VCC' voltage by measuring the internal 1.1v ref

analogRead(BANDGAPREF);

delay(10);

int refReading = analogRead(BANDGAPREF);

float supplyvoltage = (bandgap_voltage * 1024) / refReading;

logfile.print(", ");

logfile.print(supplyvoltage);

#if ECHO_TO_SERIAL

Serial.print(", ");

Serial.print(supplyvoltage);

#endif // ECHO_TO_SERIAL

logfile.println();

#if ECHO_TO_SERIAL

Serial.println();

#endif // ECHO_TO_SERIAL

digitalWrite(greenLEDpin, LOW);

// Now we write data to disk! Don't sync too often - requires 2048 bytes of I/O to SD card

// which uses a bunch of power and takes time

if ((millis() - syncTime) < SYNC_INTERVAL) return;

syncTime = millis();

// blink LED to show we are syncing data to the card & updating FAT!

digitalWrite(redLEDpin, HIGH);

logfile.flush();

digitalWrite(redLEDpin, LOW);

}