Activity 3: Appendix 17: ESP2866 Code Example

(Note: from Adafruit

https://learn.adafruit.com/esp8266-temperature-slash-humidity-webserver/code)

/* DHTServer - ESP8266 Webserver with a DHT sensor as an input

Based on ESP8266Webserver, DHTexample, and BlinkWithoutDelay (thank you)

Version 1.0  5/3/2014  Version 1.0   Mike Barela for Adafruit Industries

*/

#include <ESP8266WiFi.h>

#include <WiFiClient.h>

#include <ESP8266WebServer.h>

#include <DHT.h>

#define DHTTYPE DHT22

#define DHTPIN  2

const char* ssid     = "YourRouterID";

const char* password = "YourRouterPassword";

ESP8266WebServer server(80);

// Initialize DHT sensor

// NOTE: For working with a faster than ATmega328p 16 MHz Arduino chip, like an ESP8266,

// you need to increase the threshold for cycle counts considered a 1 or 0.

// You can do this by passing a 3rd parameter for this threshold.  It's a bit

// of fiddling to find the right value, but in general the faster the CPU the

// higher the value.  The default for a 16mhz AVR is a value of 6.  For an

// Arduino Due that runs at 84mhz a value of 30 works.

// This is for the ESP8266 processor on ESP-01

DHT dht(DHTPIN, DHTTYPE, 11); // 11 works fine for ESP8266

float humidity, temp_f;  // Values read from sensor

String webString="";     // String to display

// Generally, you should use "unsigned long" for variables that hold time

unsigned long previousMillis = 0;        // will store last temp was read

const long interval = 2000;              // interval at which to read sensor

void handle_root() {

server.send(200, "text/plain", "Hello from the weather esp8266, read from /temp or /humidity");

delay(100);

}

void setup(void)

{

// You can open the Arduino IDE Serial Monitor window to see what the code is doing

Serial.begin(115200);  // Serial connection from ESP-01 via 3.3v console cable

dht.begin();           // initialize temperature sensor

// Connect to WiFi network

WiFi.begin(ssid, password);

Serial.print("\n\r \n\rWorking to connect");

// Wait for connection

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.println("DHT Weather Reading Server");

Serial.print("Connected to ");

Serial.println(ssid);

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handle_root);

server.on("/temp", [](){  // if you add this subdirectory to your webserver call, you get text below :)

gettemperature();       // read sensor

webString="Temperature: "+String((int)temp_f)+" F";   // Arduino has a hard time with float to string

server.send(200, "text/plain", webString);            // send to someones browser when asked

});

server.on("/humidity", [](){  // if you add this subdirectory to your webserver call, you get text below :)

gettemperature();           // read sensor

webString="Humidity: "+String((int)humidity)+"%";

server.send(200, "text/plain", webString);               // send to someones browser when asked

});

server.begin();

Serial.println("HTTP server started");

}

void loop(void)

{

server.handleClient();

}

void gettemperature() {

// Wait at least 2 seconds between measurements.

// if the difference between the current time and last time you read

// the sensor is bigger than the interval you set, read the sensor

// Works better than delay for things happening elsewhere also

unsigned long currentMillis = millis();

if(currentMillis - previousMillis >= interval) {

// save the last time you read the sensor

previousMillis = currentMillis;

// Reading temperature for humidity takes about 250 milliseconds!

// Sensor readings may also be up to 2 seconds 'old' (it's a very slow sensor)

humidity = dht.readHumidity();          // Read humidity (percent)

temp_f = dht.readTemperature(true);     // Read temperature as Fahrenheit

// Check if any reads failed and exit early (to try again).

if (isnan(humidity) || isnan(temp_f)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

}

}